Northumbria University

15

EN0627 Report

Microcontroller Based Distance Measurement System

Adam Brown 10015647

Mitchell Smith 10017351

Contents

Introduction	1

Aims	1

Background	1

Specification	1

Team Contributions	1

Theory	2

Polaroid 6500	2

Startup	2

Operation	2

Pulse Generation Time	2

Time of Response between INIT and ECHO	2

Radius of Curvature	3

Calculating Result	3

Passband and Tolerance	4

Hardware	5

Circuit Design	5

Power Supply	5

PIC	6

Display	7

LEDs	7

Buttons	7

External Connector	7

PCB	8

Enclosure	8

Power Testing	9

Costings	9

Software	10

Clock Speed	10

Hardware Assignment	10

PORTA	10

PORTB	10

Tools	10

Techniques	11

Handling Numbers	11

Directives	11

Macros	11

Functions	12

Startup	12

Menu	13

LCD	14

Scan	16

Numerical Processing	17

Testing	17

Software Debugging	17

Stimulus	18

Experiment	18

Oscilloscope Traces	20

Analysis of Results	21

Conclusions	21

Further Development	21

Debouncing	21

Error Checking	22

Saving Results to EEPROM	22

Device-PC Interface	22

Bibliography	22

Appendix A: Build Output	1

Appendix B: Program Listing	1

Appendix C: .map File	1

# Introduction

## Aims

The aim of the project was to design, prototype, and develop an ultrasonic sensor that met the specification with software written entirely in assembly.

## Background

TRISK, a Sunderland-based company, design and manufacture infra-red paint curing equipment for the automotive industry. They are currently replacing old control electronics and mechanical timers, and switches, with PIC microcontrollers. One upgrade under consideration is a distance indicator, since the distance of the infra-red heaters is critical for performance.

## Specification

- Must use a 9V DC battery

- The PIC must use the sleep function

- Maximum power consumption: 500mW

- Maximum cost: £12

- Maximum enclosure size: 100mm×50mm×25mm

- Radius of curvature of panel detected: 100mm

- Dynamic passband range: 500mm−1300mm

- Accuracy: 97.5%

- PCB design to industrial guidelines

- Unit must withstand overspray

- Unit must indicate when close to passband

- Unit must indicate when infra-red heater is too close

## Team Contributions

The project was divided into tasks for each team member. The responsibility matrix is below.

Table 1 (Continues on next page)

|  | Adam Brown | Mitchell Smith |
| --- | --- | --- |
| Hardware Development | Primary | Secondary |
| Circuit Design | Primary | Secondary |
| Circuit Testing | Primary | Secondary |
| PCB design | Primary | Secondary |
| PCB construction | Primary | Secondary |
| Component selection | Primary | Secondary |
| Costing | Primary | Secondary |
| Software Development | Secondary | Primary |
| Software design | Secondary | Primary |
| Software debugging | Secondary | Primary |
| Startup | Secondary | Primary |
| Menu | Secondary | Primary |
| Scan | Secondary | Primary |
| Numerical Processing | Secondary | Primary |
| LCD Control | Primary | Secondary |

# Theory

## Polaroid 6500

### Startup

### Operation

- The PIC sets INIT

- The Polaroid 6500 holds ECHO low and generates 16 ultrasonic pulses

- The Polaroid 6500 waits for those pulses to return

- The Polaroid 6500 releases ECHO (goes high again)

The distance between INIT being set and ECHO going high again is the distance between the pulses and an object

Figure 1 (Polaroid, p. 3)

### Pulse Generation Time

When INIT is set, the Polaroid 6500 generates 16 pulses at 49.4kHzCITATIONPol15
            
            Polaroid,p.3,equivalent to 324μs. Equation 1 shows how this was calculated.

Equation 1

1649.4×103=324μs

### Time of Response between INIT and ECHO

The ultrasonic pulses travel at 1.8ms' to an object and back (Polaroid, p. 3) to the object and back. There are304.8mmperfoot. Therefore, there are 5.9μsmm forward and back. Equation 2 shows how this was calculated and Equation 3 showsmmμs.

Equation 2

1.8304.8=5.9μsmm

Equation 3

304.81.8=0.1693mmμs

### Radius of Curvature

When an ultrasonic pulse hits a surface, it reflects back away from it. For an object with a surface at 90°, the pulses are reflected straight back at the Polaroid 6500.

Figure 2 is a diagram showing how ultrasonic pulses are reflected off a surface at 90°. It is clear that each of the pulses will return.

Figure 3 is a diagram showing how ultrasonic pulses are reflected off a perfect circle. The pulses are reflected off the object at different angles so it is not guaranteed that they will return.

Polaroid 6500

Ultrasonic pulses

Figure 2

Polaroid 6500

Figure 3

## Calculating Result

The result is calculated in 4 steps. These are Equation 4Equation 7.

| Equation | Description |
| --- | --- |
| Equation 4 | Subtract PULSEGEN (calibration error in terms of us) from result |
| Equation 5 | Multiply by distance per us. Distance per us is a fraction, so it is scaled by 210, to become 173 |
| Equation 6 | Divide result by 210 to eliminate scaling factor |
| Equation 7 | Subtract LATENCY (calibration error in terms of mm) |

Equation 4

A=CCP1−PULSEGEN

Equation 5

USB=A×DIST

Equation 6

C=B1024

Equation 7

MMRESULTC−LATENCY

## Passband and Tolerance

The passband is the range in which the Green LED is lit. This corresponds to the ideal distance between a car and the infra-red heater. The tolerance band corresponds to distances “close” to the passband, but not within it, and lights the Amber LED.

Pass band

[FIGURE] below is a diagram showing how the passband and tolerance bands work.

Tolerance band

Fail

Fail

Figure 4

# Hardware

## Circuit Design

Figure 5

### Power Supply

The device power is supplied by a 9V PP3 size battery, this battery is held in place inside the casing by metal clip attached to the casing. Power is supplied through a battery clip wired in series with a switch; this is the main power on/off switch for the device. The power supply wires are soldered on to the PCB through holes; this allows the battery to clip into the case beneath the PCB.

The 9V power from the battery supplies a 5V 7805 series linear voltage regulator that provides power for the devices mounted on the PCB, there is a 100nF decoupling capacitor connected in parallel across the output as this is required by the regulator for proper operation. The specific voltage regulator used has a minimum voltage specification of 7V, this is sufficient to ensure the device will be usable throughout the normal voltage range of a typical 9V alkaline battery.

Power is supplied through the 9 pin connector to the ultrasound unit. The unit draws 100mA of current when not transmitting and 2000mA while transmitting at a voltage of 5V. As the current drawn when transmitting exceeds the current capabilities of the battery, an input capacitor is needed to meet peak current demands. The unit transmits 16 pulses at a frequency of 49.2kHz, giving a total transmit time of 325µs. The minimum voltage required by the ultrasound is 4.5V, so an input capacitor must not let the voltage drop more than 0.5V. 2A at 5V gives an input resistance of 2.5Ω.

Given this information, a required capacitance value can be calculated.

Vt=e−tRC

C=−t×RlnVt

C=−325μs×2.5Ωln0.5V=1172µF

A capacitor with a capacitance of at least 1172µF is needed, as the next biggest commonly available capacitance is 1500 µF, this value was chosen for the input capacitor after researching costs.

### PIC

The microcontroller chosen was the Microchip PIC 16F1847, this is an 18 pin 8 bit PIC with 16 I/O pins available. This PIC was chosen as it had the features needed for the device, could be obtained at a good price, and could be programmed using the hardware available.

Initial research into the model of PIC to be used identified the PIC 16F1707 as the ideal PIC for the device due to its cost and features suited to the design. However as this PIC has only been recently released, the programming boards available did not support this model, and the PIC16F1847 was chosen as it was the second best in performance for price identified through research. (Microchip Technology, 2013b)

The 16F1847 has 16 I/O pins split over two ports (A and B) consisting of 15 pins configurable as inputs or outputs and one input only pin on port A. Pin allocation was decided based on the available features on both ports, as the Capture and Compare pins are located on port B, this port was reserved for the ultrasound interface. As the LCD display requires 7 pins for its operation, port A was reserved for this purpose as it has 7 bidirectional I/O pins. (Microchip Technology, 2013a)

To ensure easy removal and replacement of the PIC for external programming or in case it should develop a fault, the PIC was mounted on an 18 pin DIL socket. This allows the PIC to be removed without the need to de-solder which could result in damage to the rest of the board.

An In Circuit Serial Programming (ISCP) header is provided on the board for programming the PIC in situ. These pins interface with the appropriate pins on the pic to allow the use of an external programmer following the ISCP specification (Microchip Technology, 1997). The pins used for programming are the power supply pins (VDD and VSS) the programming voltage pin (VPP) and the ICSP Clock and Data lines (RB6 and RB7). The VPP pin is held high by a 10kΩ resistor.

### Display

The device makes use of an 8 character wide, 2 line LCD display to display information to the user. This information includes the current distance setting, options to increment and decrement the distance, measured distance and if the measured distance lies within the set tolerance.

The display used is the Powertip PC1602ARU, this display is a reflective Liquid Crystal Display (LCD) with no backlight. The decision to use a non-backlit display was made as backlit LCDs are more expensive and use considerably more power. The LCD controller is powered by a 5V supply and the LCD display by a voltage up to 5V, varying this voltage alters the contrast of the display and is achieved by connecting a resistor in series to ground from the VE (Contrast Voltage) pin. A resistance value of 2.2kΩ was found to provide an acceptable contrast level.

The LCD chosen uses the Hitachi HD44780 LCD controller which provides a standardised set of instructions for the LCD, the inputs consist of eight data lines (DB0 – DB7) and three control lines (RS, R/W and EN). The data lines are configured as outputs when R/W is held high, this is used to read a busy flag output on DB7 to indicate when the LCD controller can accept data or commands. Due to the number of I/O lines available on the microcontroller, the LCD controller was configured in four bit mode using only data lines DB4 – DB7, this is set during the initialisation of the controller.

The unused data lines (DB0 – DB3) are connected to ground to ensure they do not float and cause unintended behaviour of the LCD controller. The LCD board is connected to the main circuit board by header pins.

The LCD has an initialisation delay at power on which is slower than the controlling PIC so a delay has been introduced in the PIC initialisation to ensure the LCD is ready to accept commands when the PIC starts transmitting.

### LEDs

The device includes red, amber and green LEDs to indicate if an object is within an acceptable range of the device. The red LED indicates it is out of the set range, the amber that it is close to the set range, and the green that it is in range.

The anodes of the LEDs are connected to output pins on the microcontroller and the cathodes to ground through 470Ω current limiting resistors. The resistors limit the maximum current drawn per LED to 11mA which is below the microcontrollers maximum rated current per pin of 25mA. The measured forward bias voltages for the LEDs were 2.1V, 2.3V and 2.2V for the red, amber and green LEDs respectively; this gives a power level of 23 – 25 mW per LED which provides an acceptable level of brightness for the LEDs to act as indicators.

### Buttons

The device uses buttons to navigate the menus and set options, three buttons are used, for left, right and enter functions. The buttons are SPST push-to-make type, and are connected from the PIC input pins to ground. The internal pull up functionality of port B is used as this eliminates the need for external pull up resistors.

### External Connector

The connection to the ultrasound unit is made through a 9 pin DB9 size connector. The connector on the ultrasound is of the female type, so the decision was made to include a male type connector on the device, this will protrude from the side of the enclosure. The pins map directly so a straight-through type cable can be used.

The external connector carries power for the ultrasound unit so that no external power connections are required.

## PCB

Figure 6

The circuit diagram was used to design a PCB for all the components. The designed PCB is a two layer, through hole design with top and bottom layers. The components were arranged to fit and then the tracks were routed to create connections, because of the dimensions of the LCD display, the rest of the circuitry has limited space and so is designed to be compact.

Holes are drilled into the PCB to mount components; these holes also provide electrical connections on both sides of the PCB which the components will be soldered into. As the PCB is a two layer design, special via holes are used to allow a track to cross from one side to the other, these must be drilled and filled with solder to function.

The PCB is sized to fit exactly into the enclosure chosen taking into account the space taken up by the lid screw mounts, this ensures all the available space is used. The 9 pin connector overhangs the right side of the PCB as it will protrude from the enclosure to enable the cable to be connected.

## Enclosure

The enclosure used for the device is an ABS plastic type enclosure with a lid secured with four screws. This type of enclosure was chosen due to its low cost and the ease in which it can be machined due to the type of plastic.

The amount an enclosure protects against solid and liquid ingress is measured by its IP rating, with the first digit corresponding to solid ingress and the second to liquid. The enclosure chosen is IP54 rated, this means that the enclosure is sealed with limited dust protection and protection from water spray from any direction. This is important as the device must be protected from overspray and be able to be wiped clean.

As the enclosure will be modified to allow for components to protrude, any gaps resulting will be sealed using hot melt glue to preserve the ingress protection.

## Power Testing

The power consumption of the board with and without the ultrasound unit attached is given in the table below.

Table 2

| Configuration | Voltage (V) | Current (mA) | Power (mW) |
| --- | --- | --- | --- |
| Board Only (idle) | 4.931 | 1.340 | 6.608 |
| Board with Ultrasound (idle) | 4.931 | 39.50 | 194.8 |

These results show that the majority of the power consumption is due to the ultrasound device. From these figures, a typical fully charged 9V PP3 battery with a capacity of 565 mAh will last for approximately 422 hours with the ultrasound disconnected and approximately 13.3 hours with the ultrasound connected. (Duracell)

## Costings

Components were sourced online from Farnell, prices for a single unit and bulk purchase for 10 units are given. The highest cost item was the LCD display; this has been justified as it enhances the usability of the devices by displaying information clearly. The cost of the device is under the maximum cost set in the specification of £12.

Table 3

| Component | Quantity | Model | Price (Single) | Price (10) |
| --- | --- | --- | --- | --- |
| LCD | 1 | POWERTIP PC1602ARU | £4.3300 | £3.7400 |
| PIC | 1 | PIC16F1847 | £1.8200 | £1.1700 |
| Enclosure | 1 | Deltron Enclosure | £1.0400 | £0.9580 |
| Connector | 1 | D-SUB, PLUG, THT, R/A, 9WAY | £0.8500 | £0.8500 |
| Buttons | 3 | SWITCH, SPST, 0.05A, 12VDC, THT | £0.5190 | £0.5190 |
| Voltage Regulator | 1 | MC7805ACTG | £0.4780 | £0.3330 |
| On Switch | 1 | SWITCH, ROCKER, DPST, ON-OFF | £0.4590 | £0.4590 |
| LCD Header | 1 | HEADER, 16WAY, 1ROW | £0.4240 | £0.3820 |
| Battery Leads | 1 | Battery Strap | £0.2560 | £0.2560 |
| Power Capacitor | 1 | CAP, ALU ELEC, 1500UF, 16V, RAD | £0.1960 | £0.1960 |
| Battery Holder | 1 | Battery Retainer | £0.1630 | £0.1630 |
| ICSP Header | 1 | PIN HEADER, 5POS, 5MM | £0.1520 | £0.1520 |
| DIP Socket | 1 | DIP Socket | £0.1270 | £0.1270 |
| Switch Cap | 3 | CAP, ROUND, ORANGE | £0.1233 | £0.1233 |
| Amber LED | 1 | 5MM, YELLOW, 30MCD, 588NM | £0.0643 | £0.0578 |
| Green LED | 1 | 5MM, GREEN, 100MCD, 568NM | £0.0643 | £0.0578 |
| Red LED | 1 | 5MM, RED, 40MCD, 617NM | £0.0643 | £0.0578 |
| Supply Capacitor | 1 | 0.1uF Cap | £0.0225 | £0.0192 |
| LED Resistors | 3 | CARBON , 470R, 0.25W, 5% | £0.0135 | £0.0135 |
| Total (Per Unit) |  |  | £11.1662 | £9.6344 |

# Software

The software was written entirely in PIC assembly. This helps to ensure that the code is efficient and the developer has very precise control of the software’s wall-clock time (time it takes to execute a function).

## Clock Speed

The PIC uses a 4MHz system clock. Operating at 5V, the typical current drawn by the PIC at 5V is 1.55mA (Microchip Technology Inc, p. 315), compared to the current drawn by the Polaroid 6500, this is very small, and guarantees that the program will operate quickly and can accurately measure the time between setting INIT and seeing a rising edge on ECHO.

## Hardware Assignment

### PORTA

The LCD uses 7 pins; 4 data pins (DB 4 – 7), Enable, Register Select and Read/Write; these were assigned to PORTA because implementing the algorithms necessary to communicate with the LCD were much more straightforward assuming all the pins were on a single port.

RA5 was not used because it is input only.

### PORTB

Because ECHO is connected as the CCP1 pin, it was necessary to place it on PORTB. It was assigned to PORTB.0, but it could have also been assigned to PORTB.3

Because the PIC sleeps when idle, the buttons needed to trigger an interrupt when pressed; for this reason they were assigned to PORTB, since this offers access to the Interrupt on Change feature.

The LEDs were assigned to leftover ports.

## Tools

#### IDE

We used the MPLABX development environment, and the MPASM toolkit to build the project.

The version information for the copy of MPLABX used is in Figure 7.

Product Version: MPLAB X IDE v2.05

Java: 1.7.0_25; Java HotSpot(TM) 64-Bit Server VM 23.25-b01

System: Windows 7 version 6.1 running on amd64; Cp1252; en_GB (mplab)

Figure 7

## Techniques

### Handling Numbers

The PIC is little-endian, this means that the most significant value of a number is represented by the largest bit in a register. This is crucial because it will affect how mathematical functions are developed, eg a quick divide is possible when the denominator is a power of2n, because this is equivalent to a bit shift. In a little-endian system the register must be shifted right n places, but in a big-endian system the register would be shifted n places to the left.

Little-endianness is illustrated by Table 4.

Table 4

| Decimal | Binary | Register (memory location = k) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 0000 1010 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
|  |  | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |

#### 16-Bit Numbers

16-bit numbers place the upper byte of data into the largest memory location, and the lower byte of data into the smallest memory location.

This is illustrated by Table 5

Table 5

| Decimal |  | Binary | Upper register ( memory location = k + 1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40970 | Upper byte | 1010 0000 | b7 | b6 | b5 | b4 |  | b3 | b2 |  | b1 |  | b0 |  |
|  |  |  | 1 | 0 | 1 | 0 |  | 0 | 0 |  | 0 |  | 0 |  |
|  | Lower byte | 0000 1111 | Lower register (memory location = k) |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | b7 | b6 | b5 | b4 | b3 |  |  | b2 |  | b1 |  | b0 |
|  |  |  | 0 | 0 | 0 | 0 | 1 |  |  | 1 |  | 1 |  | 1 |

It was essential to standardise the handling of 16-bit files to ensure that all files are handled correctly by the 16-bit numerical processing functions

### Directives

Directives are labels that the assembler recognises and substitutes for lines of code. They help make code human-readable.

### Macros

Macros generate code in locations where their respective labels are used. There is a tradeoff between using macros or using the call function and indirect registers: macros require more program memory, but call functions would mean passing files to and fro using the indirect registers (eg FSR0, FSR1, X, Y)

The principal advantage of using macros is improving the human-readability of the code; an action that may require five lines of code can be substituted for one. The point is illustrated in Figure 8.

| Program Listing (pre-assembly) | Deassembly (post-assembly) |
| --- | --- |
| movff MACRO f1,f2 banksel f1 movf f1,W banksel f2 movwf f2 ENDM org 0x00 movff fA, fB end | org 0x00 banksel fA movf fA,W banksel fB movwf fB end |

Figure 8

Table 6

| Macro | Description |
| --- | --- |
| movlf | Moves 8-bit literal k into file f1 |
| movlf16 | Moves 16-bit literal k into file f1 |
| movff | Copies 8-bit file f1 into 8-bit file f2 |
| movff16 | Copies 16-bit file f1 into 16-bit file f2 |
| clear | Sets the contents of 8-bit file f1 to 0x00 |
| movfi | Copies the contents of 8-bit file f1 into indirect register fsrn |
| movif16 | Copies the contents of fsrn and fsrn+1 into 16-bit register f1 |
| movli | Moves literal l1 into indirect register fsrn |
| movli16 | Moves 16-bit literal l1 into the register pointed to by the contents of fsrn and fsrn+1 |
| movif16 | Moves the 16-bit value contained in the address located in fsrn and fsrn+1 into 16-bit register f1 |
| get_ascii_ln | Converts the lower nibble (b3-b0)of the argument into an ASCII number between “0” and “9”. |
| get_ascii_un | Converts the upper nibble (b7-b4) of the argument into an ASCII character between “0” and “9” |

## Functions

### Startup

Startup initialises registers and the pinout. Includes a 5ms delay to ensure that the Polaroid 6500 is initialised after powerup

[Table] below shows the functions Startup offers

Table 7

| Label | Description of code |
| --- | --- |
| Setup | Initialises the PIC and includes 5ms delay to ensure Polaroid 6500 is initialised before it can be operated |

### Menu

The menu controls which function is called when ENTER is pressed, and cycling through these functions.

The menu set of functions includes those for altering the size of the passband and tolerance band.

Pressing enter makes the PIC jump to the location stored in menu_pointer. When the user moves to a different menu, the location stored in menu_pointer is updated.

[TABLE] below shows the functions Menu offers

Table 8

| Label | Description of code |
| --- | --- |
| cycle_left | Cycle left increments the menu_selected register and then error checks to ensure it does not overflow. If it does then the minimum value of menu_selected for that menu is copied into it. |
| cycle_right | Cycle left decrements the menu_selected register and then error checks to ensure it does not underflow. If it does then the mmaximum value of menu_selected for that menu is copied into it. |
| start_menu | Copies the value of menu_selected to W and uses this to branch to the option that the user selected. |
| upper_passband_menu | Copies the value of menu_selected to W and uses this to branch to the option that the user selected. |
| lower_passband_menu | Copies the value of menu_selected to W and uses this to branch to the option that the user selected. |
| upper_tolerance_menu | Copies the value of menu_selected to W and uses this to branch to the option that the user selected. |
| lower_tolerance_menu | Copies the value of menu_selected to W and uses this to branch to the option that the user selected. |
| add_100 | Adds 100 to the location stored in current_value_pointer. Then error checks to ensure that value does not exceed 2000mm. If it does then the value is set to 2000. |
| add_10 | Adds 10 to the location stored in current_value_pointer. Then error checks to ensure that value does not exceed 2000mm. If it does then the value is set to 2000. |
| add_1 | Adds 1 to the location stored in current_value_pointer. Then error checks to ensure that value does not exceed 2000mm. If it does then the value is set to 2000. |
| sub_100 | Subtracts 100 from the location stored in current_value_pointer. Then error checks to ensure that value does not go below 500mm. If it does then the value is set to 500. |
| sub_10 | Subtracts 10 from the location stored in current_value_pointer. Then error checks to ensure that value does not go below 500mm. If it does then the value is set to 500. |
| sub_1 | Subtracts 1 from the location stored in current_value_pointer. Then error checks to ensure that value does not go below 500mm. If it does then the value is set to 500. |

### LCD

[TABLE] below shows the functions LCD offers

Table 9

| Label | Description of Code |
| --- | --- |
| lcd_setup | Initialises the LCD |
| lcdbusy | Prints the contents of FSR0 to the LCD, increments FSR0 and loops round until it reaches a null character, then it returns. |
| lcdcmd | Passes a specified command to the LCD |
| set_lcd_top_line | Sets the LCD to the top line |
| set_lcd_bottom_line | Sets the LCD to the bottom line |
| lcd_clear | Clears the LCD screen |
| lcdoff | Switches the LCD off |
| lcdon | Switches the LCD on |
| lcd_print |  |
| update_lcd_start_menu | Copies menu_selected to W and branches to the print function for that option. |
| update_lcd_upper_passband_menu | Copies menu_selected to W and branches to the print function for that option. |
| update_lcd_lower_passband_menu | Copies menu_selected to W and branches to the print function for that option. |
| update_lcd_upper_tolerance_menu | Copies menu_selected to W and branches to the print function for that option. |
| update_lcd_lower_tolerance_menu | Copies menu_selected to W and branches to the print function for that option. |
| print_start_string | Clears the LCD and then prints start_string to the top line of the LCD |
| print_scan_string | Clears the LCD and then prints scan_string to the top line of the LCD |
| print_pass_string | Converts scan result into the equivalent ASCII characters. Clears the LCD and then prints scan_string to the top line of the LCD, then passes pass_string, the result of the scan and mm_string to the bottom line of the lcd. |
| print_tolerance_string | Converts scan result into the equivalent ASCII characters. Clears the LCD and then prints scan_string to the top line of the LCD, then passes tolerance_string, the result of the scan and mm_string to the bottom line of the lcd. |
| print_fail_string | Converts scan result into the equivalent ASCII characters. Clears the LCD and then prints scan_string to the top line of the LCD, then passes fail_string, the result of the scan and mm_string to the bottom line of the lcd. |
| print_timeout_string | Clears the LCD and then prints scan_string to the top line of the LCD, then passes timeout_string, to the bottom line of the lcd. |
| print_upper_passband_string | Clears the LCD and then prints upper_passband_string to the top line of the LCD |
| print_lower_passband_string | Clears the LCD and then prints lower_passband_string to the top line of the LCD |
| print_upper_tolerance_string | Clears the LCD and then prints lower_tolerance_string to the top line of the LCD |
| print_lower_tolerance_string | Clears the LCD and then prints lower_tolerance_string to the top line of the LCD |
| print_add_100_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints add_100_string to the top line of the LCD, then passes the ASCII to the bottom line |
| print_add_10_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints add_10_string to the top line of the LCD, then passes the ASCII to the bottom line |
| print_add_1_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints add_1_string to the top line of the LCD, then passes the ASCII to the bottom line |
| print_sub_100_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints sub_100_string to the top line of the LCD, then passes the ASCII to the bottom line |
| print_sub_10_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints sub_10_string to the top line of the LCD, then passes the ASCII to the bottom line |
| print_sub_1_string | Converts contents of register pointed to by current_value_pointer into equivalent ASCII characters. Clears the LCD and then prints sub_1_string to the top line of the LCD, then passes the ASCII to the bottom line |
| hex16_2_ascii | Converts a 16 bit file, f1 into a 4 character wide ASCII number. |

### Scan

Scan calls the Ultrasound function, which acquires a result and copies it into the X register. If a result was received, the number is processed and compared to the passband and tolerances. If there was a timeout, then the PIC jumps straight back to the end of Scan.

Scan converts the result from time into millimetres, and subtracts the pulse generation and electronic latency from the result.

Scan uses two assembler constants, PULSEGEN and LATENCY. These are fiddle factors that can correct unusual delays in the time between INIT and ECHO.

Conversion is performed using numerical processing functions (eg multiply, add, subtract). One alternative was to use a lookup table, with distances corresponding to specific values of CCP1. Although this would have been faster, it would also mean reserving a large block of program memory, and would have to be rewritten every time it is used with a sensor that is calibrated differently.

[TABLE] shows the functions Scan offers.

Table 10

| Label | Description of code |
| --- | --- |
| scan | Calls ultrasound and decides to move to scan_pass or scan_timeout depending on whether PASS or FAIL bit of usresult set |
| scan_pass | Convert result from CCP1 to distance in MM Checks if within passband, if above move to above_passband, if below move to below_passband, if within, sets PASS bit of usresult and moves to scan_end |
| below_passband | Checks if result is within lower tolerance, if within then set TOLERANCE bit of usresult and move to scan_end, otherwise move to scan_fail |
| above_passband | Checks if result is within upper tolerance, if within then set TOLERANCE bit of usresult and move to scan_end, otherwise move to scan_fail |
| scan_fail | Sets FAIL bit of usresult and moves to scan_end |
| scan_timeout | Sets TIMEOUT bit of usresult and moves to scan_end |
| scan_end | Returns |
| ultrasound | Gets result from Polaroid 6500 and returns with PASS or FAIL bit of usresult set depending on whether acquired result or timed out |
| wait_loop | Wait in loop until rising edge on ECHO pin, or TMR1 overflows |
| timeout | Sets the FAIL bit of usresult and moves to got_result |
| success | Sets PASS bit of usresult and moves to got_result |
| got_result | Clears init, CCP1CON and interrupt flags. Includes an 80ms delay using TMR0 so that the ultrasound is definitely cycled before the next scan |
| ults_end | Disable and clear TMR0 setup |

#### Ultrasound

Ultrasound uses TMR1 and CCP1 to time the response from ECHO immediately after INIT has  been set. It includes an 80ms recycle period delay[source] after acquiring the result. If TMR1 overflows (32768μs  then PIC sets usresult.TIMEOUT and returns to Scan

The function initiates a scan by setting INIT, and is cleared immediately after receiving a result or TMR1 overflow.

CCP1 is the best peripheral for capturing a result because it automatically copies the contents of TMR1 the cycle after a rising edge is seen on ECHO. This reduces the amount of code because the programmer does not have to copy the contents of TMR1 or subtract any offset between the rising edge of ECHO and copying TMR1.

### Numerical Processing

[TABLE] below shows the functions in this section.

Table 11

| Label | Description of code |
| --- | --- |
| mult16x16 | Multiplies the 16-bit file X by 16-bit register Y, and the places result into 40-bit file result |
| add16x16 | Adds the 16-bit file X by 16-bit register Y, and the places result into 40-bit file result |
| subt16x16 | Subtracts the 16-bit file Y from 16-bit register X, and the places result into 40-bit file result |
| div40bin | Divides the 40-bit file result by 1,24. This is implemented by bit shifting right 10 places. |

# Testing

## Software Debugging

Before downloading the code to a PIC, it can be simulated in MPLABX. It is possible to see the contents of the data and program memory, and as such is very useful for ironing out software bugs.

Figure 9 shows the data memory displayed during simulation. It is also possible to see the program memory, SFRs and stack.

Figure 9

### Stimulus

The stimulus is a feature that emulates signals on pins. It is useful for testing inputs, such as the buttons and the ECHO pin. Using this meant that the software could be tested independently of the hardware.

Figure 10 is a picture of the stimulus window.

Figure 10

## Experiment

The ultrasound ranging device has been tested across its operational range of 500mm to 1300mm, the results indicate that the ranging device meets the specification and that the human-device interface operates as expected.

The ultrasound module was tested using default passband and tolerance band, shown in [TABLE]

Table 12

| Band | Values |
| --- | --- |
| Passband | 875 mm – 800 mm |
| Tolerance Band | 919 mm – 760 mm |

[EQUATION] shows how the percentage accuracy was calculated.

Equation 8

Accuracy=100×measured−actualactual

[TABLE] below displays the results.

Actual results were determined using an uncalibrated meter rules. Measured values are those presented on the LCD screen after each test.

Table 13

| Actual | Measured | LED Lit | String Passed | Correct Behaviour? | % Accuracy |
| --- | --- | --- | --- | --- | --- |
| 500 | 500 | Red | "FAIL" | Y | 100.00% |
| 550 | 550 | Red | "FAIL" | Y | 100.00% |
| 600 | 600 | Red | "FAIL" | Y | 100.00% |
| 650 | 650 | Red | "FAIL" | Y | 100.00% |
| 700 | 700 | Red | "FAIL" | Y | 100.00% |
| 750 | 750 | Red | "FAIL" | Y | 100.00% |
| 760 | 760 | Amber | "FAIL" | Y | 100.00% |
| 770 | 770 | Amber | "FAIL" | Y | 100.00% |
| 780 | 780 | Amber | "FAIL" | Y | 100.00% |
| 790 | 790 | Amber | "FAIL" | Y | 100.00% |
| 800 | 800 | Green | "PASS" | Y | 100.00% |
| 810 | 810 | Green | "PASS" | Y | 100.00% |
| 820 | 820 | Green | "PASS" | Y | 100.00% |
| 830 | 830 | Green | "PASS" | Y | 100.00% |
| 840 | 840 | Green | "PASS" | Y | 100.00% |
| 850 | 850 | Green | "PASS" | Y | 100.00% |
| 860 | 860 | Green | "PASS" | Y | 100.00% |
| 870 | 870 | Green | "PASS" | Y | 100.00% |
| 880 | 880 | Amber | "FAIL" | Y | 100.00% |
| 890 | 890 | Amber | "FAIL" | Y | 100.00% |
| 900 | 900 | Amber | "FAIL" | Y | 100.00% |
| 910 | 910 | Amber | "FAIL" | Y | 100.00% |
| 920 | 920 | Red | "FAIL" | Y | 100.00% |
| 930 | 930 | Red | "FAIL" | Y | 100.00% |
| 940 | 940 | Red | "FAIL" | Y | 100.00% |
| 950 | 951 | Red | "FAIL" | Y | 99.89% |
| 1000 | 1001 | Red | "FAIL" | Y | 99.90% |
| 1050 | 1051 | Red | "FAIL" | Y | 99.90% |
| 1100 | 1101 | Red | "FAIL" | Y | 99.91% |
| 1150 | 1151 | Red | "FAIL" | Y | 99.91% |
| 1200 | 1201 | Red | "FAIL" | Y | 99.92% |
| 1250 | 1252 | Red | "FAIL" | Y | 99.84% |
| 1300 | 1302 | Red | "FAIL" | Y | 99.85% |

### Oscilloscope Traces

The below traces show the relationship between the INIT signal (yellow) sent to the ultrasound unit and the ECHO signal (green) received back.

The traces in figure 11 show the very beginning of the INIT pulse sent to the ultrasound unit, the interference clearly shows the ultrasound sending pulses and its effect on the voltage supply.

Figure 11

The traces in figure 12 show the end of the INIT pulse when an ECHO pulse is received from the ultrasound. The INIT pulse is shown to be pulled low after the PIC detects the ECHO pulse, there is an approximately 15µS delay which is within the accuracy specifications.

Figure 12

The traces in figure 13 show the whole INIT pulse and the EHO reply pulse. This trace was for a distance of 500mm which the measured pulse width of 2.95ms supports.

Figure 13

### Analysis of Results

The results show that the rangefinder correctly interprets results with a minimum of 99.85% accuracy. It may not be practical to use 1mm precision when testing at longer distances.

[FIGURE] is a graph of the results, and clearly displays the linearity of results.

(mm)

(mm)

Figure 14

# Conclusions

## Further Development

### Debouncing

The buttons occasionally bounce. This is a nuisance for the user. One solution would be to include a 1 second delay between a button being pushed and a button push being registered again.

### Error Checking

Comparing the pass and tolerance bands in the software would reduce rates of human error.

At present the PIC does not compare passband and tolerance bands, therefore it is possible for the passband to be wider than the tolerance band. The user must make sure that this is not the case.

### Saving Results to EEPROM

At the moment, all settings are stored in data memory, this means that when the power is cut, the settings are lost and must be reset to default during power up.

The EEPROM is non-volatile data memory, and would make it possible to store the settings before the device is switched off, and recover them during initialisation when the power is switched on.

### Device-PC Interface

At the moment the user has to be physically close to the device, to press buttons. One future development would be to add serial over USB support so that a user could control the sensor from a workstation. It would also be possible to log data to a file on the workstation.

This could increase the number of applications of the device.

# Bibliography

Harrington, P., & Dai, D. (2014, 12 18). Email conversation. (M. Smith, Interviewer)

Microchip Technology Inc. (n.d.). Retrieved 01 29, 2015, from Microchip Technology Inc Website: http://ww1.microchip.com/downloads/en/DeviceDoc/41453B.pdf

Northumbria University. (n.d.). Coursework Assessment Specification: EN0627 2014-2015.

Polaroid. (n.d.). Retrieved 01 29, 2015, from Robot Store (HK) Website: http://www.robotstorehk.com/6500.pdf

DURACELL. MN1604. Available: http://ww2.duracell.com/media/en-US/pdf/gtcl/Product_Data_Sheet/NA_DATASHEETS/MN1604_6LR61_US_CT.pdf.

MICROCHIP TECHNOLOGY. 1997. In-Circuit Serial Programming™ (ICSP™).

MICROCHIP TECHNOLOGY. 2013a. PIC16(L)F1847 Datasheet.

MICROCHIP TECHNOLOGY. 2013b. Recently Released Products - Microchip Technology Inc. [Online]. Available: http://www.microchip.com/ParamChartSearch/chart.aspx?branchID=1013 [Accessed 28/01/2015.

# Appendix A: Build Output

make -f nbproject/Makefile-default.mk SUBPROJECTS= .build-conf

make[1]: Entering directory `D:/en0627_assignment.X

make  -f nbproject/Makefile-default.mk dist/default/production/en0627_assignment.X.production.hex

make[2]: Entering directory `D:/en0627_assignment.X

"C:\Program Files (x86)\Microchip\MPLABX\mpasmx\mpasmx.exe" -q -p16f1847 -l"build/default/production/en0627_assignment.lst" -e"build/default/production/en0627_assignment.err" -o"build/default/production/en0627_assignment.o" "en0627_assignment.asm"

Warning[210] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 104 : Extra ","

Warning[210] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 105 : Extra ","

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 279 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 280 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 281 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 282 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 283 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 284 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 285 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 286 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 290 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 292 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 293 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 294 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 295 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 301 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 302 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 303 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 347 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 350 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 353 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 138 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 138 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1245 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1246 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1260 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1312 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[305] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1367 : Using default destination of 1 (file).

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 123 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1448 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 158 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1640 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1641 : Register in operand not in bank 0.  Ensure that bank bits are correct.

Message[302] D:\EN0627_ASSIGNMENT.X\EN0627_ASSIGNMENT.ASM 1642 : Register in operand not in bank 0.  Ensure that bank bits are correct.

"C:\Program Files (x86)\Microchip\MPLABX\mpasmx\mplink.exe"    -p16f1847  -w  -m"dist/default/production/en0627_assignment.X.production.map"   -z__MPLAB_BUILD=1  -odist/default/production/en0627_assignment.X.production.cof  build/default/production/en0627_assignment.o

MPLINK 5.01, LINKER

Device Database Version 1.18

Copyright (c) 1998-2011 Microchip Technology Inc.

Errors    : 0

MP2HEX 5.01, COFF to HEX File Converter

Copyright (c) 1998-2011 Microchip Technology Inc.

Errors    : 0

make[2]: Leaving directory `D:/en0627_assignment.X

make[1]: Leaving directory `D:/en0627_assignment.X

BUILD SUCCESSFUL (total time: 1s)

Loading code from D:/en0627_assignment.X/dist/default/production/en0627_assignment.X.production.hex...

Loading symbols from D:/en0627_assignment.X/dist/default/production/en0627_assignment.X.production.cof...

Loading completed

# Appendix B: Program Listing

D:\en0627_assignment.X\en0627_assignment.asm

1 ;*******************************************************************************

2 ; en0627_assignment.asm

3 ; Authors....: Mitchell Smith (w10017351), Adam Brown (w10015647)

4 ; Revision...: 1.01

5 ; Date.......: 2015-01-28

6 ;

7 ;   Description

8 ;

9 ; Designed to meet the specification given.

10 ; Calculates distance using result from Polaroid 6500 ultrasonic sensor

11 ; then determines if within acceptable limits and lights LEDs depending on

12 ; result

13 ;

14 ; Includes a menu that is printed to LCD for human-device interface

15 ;

16 ;   Pinout

17 ;                                      _________

18 ;                 RA2 / DB6 -| 16f1847   |- DB5 / RA1

19 ;                 RA3 / DB7 -|                 |- DB4 / RA0

20 ;                   RA4 / RS -|                  |- EN / RA7

21 ;                  RA5 / N/A -|                 |- RW / RA6

22 ;                             Vss -|                  |- Vdd

23 ;              RB0 / ECHO -|                  |- RIGHT / RB7

24 ;                 RB1 / INIT -|                  |- LEFT / RB6

25 ;       RB2 / RED_LED -|                  |- ENTER / RB5

26 ; RB3 / AMBER_LED -|_________|- GREEN_LED / RB4

27 ;

28

29

30

31     list p=16F1847

32     #include    <p16F1847.inc>

33

34 ; +++ CONSTANTS +++

35 DIST_MAX        EQU .1300 ; The maximum distance we calculate is 2000mm, 0x7D0

36 DIST_MIN        EQU .0500 ; The mimimum distance we calculate is 400mm, 0x190

37 PASSBAND_MAX    EQU .0875 ; 0x032A

38 PASSBAND_MIN    EQU .0800 ; 0x02DF

39 TOLERANCE_MAX   EQU .0919 ; PASSBAND_MIX + 5% 0x0397

40 TOLERANCE_MIN   EQU .0760 ; PASSBAND_MIN - 5% 0x02F8

41

42 MM_PER_US   EQU 0x00AE ; (mm per second * tinstruction * 1024) = .174

43 PULSEGEN    EQU 0x0000 ;0x01B8;\ The time it takes for ultrasound to generate pulses

44 LATENCY     EQU 0x0000 ;0x00C8    ;\ 200mm

45 FAIL        EQU 0

46 PASS        EQU 1

47 TOLERANCE   EQU 2

48 TIMEOUT     EQU 3

49

50 ; The maximum value of menu_selected. Must be equal to

51 ; limit = n +1 (eg options 1...3 .'. n = 3 + 1 = 4)

52 START_MENU_MAX              EQU .6

53 UPPER_PASSBAND_MENU_MAX     EQU .8

54 LOWER_PASSBAND_MENU_MAX     EQU .8

55 UPPER_TOLERANCE_MENU_MAX    EQU .8

56 LOWER_TOLERANCE_MENU_MAX    EQU .8

57

58 ; The minimum value of menu_selected for menus.

59 START_MENU_MIN              EQU .2

60 LIMIT_MENU_MIN              EQU .2

61 UPPER_PASSBAND_MENU_MIN     EQU .2

62 LOWER_PASSBAND_MENU_MIN     EQU .2

63 TOLERANCE_MENU_MIN          EQU .2

64 UPPER_TOLERANCE_MENU_MIN    EQU .2

65 LOWER_TOLERANCE_MENU_MIN    EQU .2

66

67 ; Hardware Assignment

68 LCDPORT EQU PORTA

69 LCDDIR  EQU TRISA

70 DB4     EQU .0

71 DB5     EQU .1

72 DB6     EQU .2

73 DB7     EQU .3

74 RS      EQU .4

75 RW      EQU .6

76 EN      EQU .7

77

78 USPORT  EQU PORTB

79 USDIR   EQU TRISB

80 USPU    EQU WPUB

81 ECHO    EQU .0 ; RB.0 (Input, internal pullup)

82 INIT    EQU .1 ; RB.1 (Output)

83

84 LEDPORT     EQU PORTB

85 LEDDIR      EQU PORTB

86 RED_LED     EQU .2 ; RB.5

87 AMBER_LED   EQU .3 ; RB.6

88 GREEN_LED   EQU .4 ; RB.7

89

90 BUTTONPORT  EQU PORTB

91 BUTTONDIR   EQU TRISB

92 BUTTONPU    EQU WPUB

93 ENTER       EQU .5 ; RB.4 (Input, internal pullup)

94 LEFT        EQU .6 ; RB.2 (Input, internal pullup)

95 RIGHT       EQU .7 ; RB.3 (Input, internal pullup)

96

97 ; LCD

98 #define     LCDMASK     0x0F

99 #define     STRH        0x00

100 #define     STRL        0x20

101

102 ; Data Memory

103     cblock 0x20

104     current_menu:1, menu_selected:1, menu_pointer:2, lcd_pointer:2, distance:2,

105     menu_max:1, menu_min:1, upper_tolerance:2, lower_tolerance:2,

106     upper_passband:2

107     endc

108     cblock 0x30

109         lower_passband:2, X:2, Y:2, BCD:4 ,result:5, count:1

110     endc

111     cblock  0x40

112         temp:1,usresult:1,ascii:6, current_value_pointer:1

113     endc

114     cblock      0x70

115         deltime:1, cmdbyte:1

116     endc

117

118 ; Macros

119 ; Moves l into f

120 movlf   MACRO   ell,eff

121         banksel eff

122         movlw   ell

123         movwf   eff

124         ENDM

125

126 ; Moves 16bit l into 16bit f

127 movlf16 MACRO   ell,eff

128         banksel eff

129         movlw   low ell

130         movwf   eff

131         movlw   high ell

132         movwf   eff+1

133         ENDM

134

135 ; Moves 8bit f1 into 8bit f2

136 movff   MACRO   f1,f2

137         banksel f1

138         movf    f1,W

139         banksel f2

140         movwf   f2

141         ENDM

142

143 ; Moves 16bit f1 into 16bit f2

144 movff16 MACRO   f1,f2

145         banksel f1

146         movf    f1,W

147         banksel f2

148         movwf   f2

149         banksel f1+1

150         movf    f1+1,W

151         banksel f2

152         movwf   f2+1

153         ENDM

154

155 ; Selects and clears f1

156 clear   MACRO   f1

157         banksel f1

158         clrf    f1

159         ENDM

160

161 ; Moves f into FSRn

162 movfi   MACRO   f1,fsrn

163         banksel f1

164         movf    f1,W

165         movwi   0[fsrn]

166         ENDM

167

168 ; Loads fsrn and fsrn+1 into a 16-bit file

169 movif16 MACRO   fsrn,f1

170         moviw   0[fsrn]

171         banksel f1

172         movwf   f1

173         moviw   1[fsrn]

174         movwf   f1+1

175         ENDM

176

177 ; Moves l into fsrn

178 movli   MACRO   ell,fsrn

179         movlw   ell

180         movwi   0[fsrn]

181         ENDM

182

183 ; Moves 16-bit l into fsrn and fsrn+1

184 movli16 MACRO   ell,fsrn

185         movlw   low ell

186         movwi   0[fsrn]

187         movlw   high ell

188         movwi   1[fsrn]

189         ENDM

190

191 ; Moves 0[FSR0] and 1[FSR0] into f1 and f1+1

192 moviw16 MACRO   fsrn,f1

193         moviw   0[fsrn]

194         banksel f1

195         movwf   f1

196

197         moviw   1[fsrn]

198         movwf   f1+1

199         ENDM

200

201 ; Returns the lower nibble of a file as ASCII character using lookup table

202 get_ascii_ln    MACRO   f1,f2

203                 banksel f1

204                 movf    f1,W

205                 andlw   0x0F

206                 call    get_ascii

207                 banksel f2

208                 movwf   f2

209                 ENDM

210

211 ; Returns the upper nibble of a file as ASCII chaarcter using lookup table

212 get_ascii_un    MACRO   f1,f2

213                 banksel f1

214                 swapf   f1,W

215                 andlw   0x0F

216                 call    get_ascii

217                 banksel f2

218                 movwf   f2

219                 ENDM

220

221 ; Subtract l from f, place in w

222 subfl_to_w      MACRO   l1,f1

223                 banksel f1

224                 movlw   l1

225                 subwf   f1,w

226                 ENDM

227

228 ;*******************************************************************************

229 ; Program

230 ;*******************************************************************************

231

232     org  0x00

233     goto setup

234     org 0x04

235 ISR

236 ;****************************

237 ;   Strings

238 ;****************************

239 start_string              dt    "START MENU\0"

240 scan_string               dt    "SCAN\0"

241 testing_string            dt    "TESTING\0"

242 upper_passband_string     dt    "UPPER PSBND\0"

243 lower_passband_string     dt    "LOWER PSBND\0"

244 upper_tolerance_string    dt    "UPPER TOL\0"

245 lower_tolerance_string    dt    "LOWER TOL\0"

246 pass_string               dt    "PASS: \0"

247 fail_string               dt    "FAIL: \0"

248 timeout_string            dt    "USOUND TIMEOUT\0"

249 mm_string                 dt    " MM\0"

250 add_100_string            dt    "ADD 100\0"

251 add_10_string             dt    "ADD 10\0"

252 add_1_string              dt    "ADD 1\0"

253 sub_100_string            dt    "SUB 100\0"

254 sub_10_string             dt    "SUB 10\0"

255 sub_1_string              dt    "SUB 1\0"

256

257 ;****************************

258 ;   Setup

259 ;

260 ; Contains code for initialisation of PIC and ultrasound

261 ; Only ever runs once, when PIC started up

262 ;

263 ; Initialises the system clock, pinout, LCD and waits 5ms to ensure Polaroid

264 ; 6500 intialised.

265 ;****************************

266 setup

267         ; use 4Mhz fosc

268         movlf   0xEB,OSCCON

269

270         movlf   0xF0,TMR0

271         bsf     INTCON,TMR0IE ; Set TMR0 to count 5ms w/ pre of 4 and use Finst

272         movlf   0x01,OPTION_REG

273

274         ; Make sure no PORTB pins are analogue pins

275         clear   ANSELB

276

277         ; Initialise pin direction

278         clear   TRISB           ;\ Set buttons as inputs

279         bsf     TRISB,LEFT      ;|

280         bsf     TRISB,RIGHT     ;|

281         bsf     TRISB,ENTER     ;/

282         bcf     TRISB,GREEN_LED ;\ Set LED pins as outputs

283         bcf     TRISB,AMBER_LED ;|

284         bcf     TRISB,RED_LED   ;/

285         bcf     TRISB,INIT      ; Set INIT as output

286         bsf     TRISB,ECHO      ; Set ECHO as input

287

288         ; Switch CCP1 from RB3 to RB0

289         banksel APFCON0

290         bsf     APFCON0,CCP1SEL

291         clear   WPUB            ;\Set pullups for buttons, will use

292         bsf     WPUB,LEFT      ;| -ive edge triggering

293         bsf     WPUB,RIGHT     ;|

294         bsf     WPUB,ENTER     ;/

295         bsf     WPUB,ECHO

296         ; Initialise the LCD

297         call    lcd_setup

298

299         ; Enable -ive edge interrupts on buttons

300         banksel IOCBN

301         bsf     IOCBN,LEFT

302         bsf     IOCBN,RIGHT

303         bsf     IOCBN,ENTER

304

305         ; Wait 5ms so that Polaroid 6500 is definitely initialised

306         btfss   INTCON,TMR0IF

307         goto    $-1

308

309         ; Disable TMR0 and clear IF

310         bcf     INTCON,TMR0IE

311         bcf     INTCON,TMR0IF

312         clear   OPTION_REG

313

314         ; Initialise variables

315         movlf16   TOLERANCE_MIN,lower_tolerance

316         movlf16   TOLERANCE_MAX, upper_tolerance

317         movlf16   PASSBAND_MIN, lower_passband

318         movlf16   PASSBAND_MAX, upper_passband

319

320         ; Initialise the menu

321         goto    set_start

322

323 ;****************************

324 ;   Main

325 ;

326 ; Program loop

327 ; Default state is sleep, woken when a button is pressed

328 ;

329 ; Will then move to perform action appropriate for the button pressed

330 ;

331 ; When that action is finished, PIC moves into update_lcd,

332 ;****************************

333 main

334         bsf INTCON,IOCIE

335         sleep             ; Wait for input

336

337         ; TMR0 sometimes overflows, not sure why yet but this stops it from

338         ; waking the PIC up

339         btfsc   INTCON,TMR0IF

340         goto    TMR0_reset

341

342         ; Disable IOC pins

343         clear   PIR1

344         bcf     INTCON,IOCIE

345

346         banksel IOCBF

347         btfsc   IOCBF,LEFT

348         goto    cycle_left

349

350         btfsc   IOCBF,RIGHT

351         goto    cycle_right

352

353         btfsc   IOCBF,ENTER

354         goto    goto_menu

355

356 TMR0_reset

357         bcf     INTCON,TMR0IF

358         goto    main

359

360 ; Jump to location stored in menu_pointer

361 goto_menu

362     banksel menu_pointer

363     movff    menu_pointer+1,PCLATH

364     bsf      PCLATH,7

365     movff    menu_pointer,PCL

366

367 ; Cycles "left" through menu, error checks to make sure never go above max limit

368 cycle_left

369     banksel menu_selected

370     incf menu_selected,F

371     call error_check_upper

372     goto update_lcd

373

374 ; Cycles "right" thru menu, error checks to make sure never go above max limit

375 cycle_right

376     banksel menu_selected

377     decf menu_selected,F

378     call error_check_lower

379     goto update_lcd

380

381 ; Jump to location stored in lcd_pointer

382 update_lcd

383         movff   lcd_pointer+1,PCLATH

384         movff   lcd_pointer,PCL

385

386 ; Get ready to jump back to main

387 fin

388         clear IOCBF

389         goto main

390

391 ; Check if menu_option is less than minimum value for current menu, if so then

392 ; copies minimum value into menu_option

393 error_check_lower

394     banksel menu_min

395     movf   menu_min,W

396     subwf  menu_selected,W

397     movf   menu_max,W

398     btfss  STATUS,C

399     movwf  menu_selected

400     return

401

402 ; Check if menu_option is grater than maximum value for current menu, if so then

403 ; copies maximum value into menu_option

404 error_check_upper

405     banksel menu_max

406     movf   menu_max,W

407     addlw  0x01

408     subwf  menu_selected,W

409     movf   menu_min,W

410     btfsc  STATUS,C

411     movwf  menu_selected

412     return

413

414 ; Jump to here from goto_menu when in start menu and enter button pushed. Branch

415 ; to option currently selected

416 start_menu

417     movf menu_selected,W

418     brw

419     goto    software_error  ; W = 0

420     goto    software_error  ; W = 1

421     goto    perform_scan    ; W = 2

422     goto set_upper_passband ; W = 3

423     goto set_lower_passband ; W = 4

424     goto set_upper_tolerance; W = 5

425     goto set_lower_tolerance; W = 6

426

427 ; Jump to here from goto_menu when in upper passband menu and enter button

428 ; pushed. Branch to option currently selected

429 upper_passband_menu

430     movf menu_selected,W

431     brw

432     NOP ;error

433     NOP ; error()

434     goto  add_100 ; add 100 to value

435     goto  add_10  ; add 10 from value

436     goto  add_1   ; add 1 from value

437     goto  sub_100 ; subtract 100 from value

438     goto  sub_10  ; subtract 10 from value

439     goto  sub_1   ; subtract 1 from value

440     goto  set_start

441

442 ; Jump to here from goto_menu when in lower passband menu and enter button

443 ; pushed. Branch to option currently selected

444 lower_passband_menu

445     movf menu_selected,W

446     brw

447     NOP ;error

448     NOP ; error()

449     goto  add_100 ; add 100 to value

450     goto  add_10  ; add 10 from value

451     goto  add_1   ; add 1 from value

452     goto  sub_100 ; subtract 100 from value

453     goto  sub_10  ; subtract 10 from value

454     goto  sub_1   ; subtract 1 from value

455     goto  set_start

456

457 ; Jump to here from goto_menu when in upper tolerance menu and enter button

458 ; pushed. Branch to option currently selected

459 upper_tolerance_menu

460     movf menu_selected,W

461     brw

462     NOP ;error

463     NOP ; error()

464     goto  add_100 ; add 100 to value

465     goto  add_10  ; add 10 from value

466     goto  add_1   ; add 1 from value

467     goto  sub_100 ; subtract 100 from value

468     goto  sub_10  ; subtract 10 from value

469     goto  sub_1   ; subtract 1 from value

470     goto  set_start

471

472 ; Jump to here from goto_menu when in lower tolerance menu and enter button

473 ; pushed. Branch to option currently selected

474 lower_tolerance_menu

475     movf menu_selected,W

476     brw

477     NOP ;error

478     NOP ; error()

479     goto  add_100 ; add 100 to value

480     goto  add_10  ; add 10 from value

481     goto  add_1   ; add 1 from value

482     goto  sub_100 ; subtract 100 from value

483     goto  sub_10  ; subtract 10 from value

484     goto  sub_1   ; subtract 1 from value

485     goto  set_start

486

487 ; Jump to here from goto_menu when enter button pushed and "add 100" selected

488 ; Adds 100 the value located at the address stored in current_value_pointer, then error

489 ; checks to make sure it never goes above the absolute limit

490 add_100

491     movff16 current_value_pointer,FSR0

492

493     ; Add 100 to lower byte of current_value

494     MOVIW   0[FSR0]

495     addlw  .100 ; 0x64

496     MOVWI  0[FSR0]

497

498     ; If there was an overflow, add 1 to upper byte of current_value

499     MOVIW  1[FSR0]

500     btfsc  STATUS,C

501     addlw  .1

502     MOVWI  1[FSR0]

503

504

505     call   add_error_check

506     btfsc  WREG,FAIL

507     call   set_max_value

508

509     goto   update_lcd ; write the new distance to the lcd

510

511 ; Jump to here from goto_menu when enter button pushed and "add 10" selected

512 ; Adds 10 to the value located at the address stored in current_value_pointer, then error

513 ; checks to make sure it never goes above the absolute maximum limit

514 add_10

515     movff16 current_value_pointer,FSR0

516

517     ; Add 10 to lower byte of current_value

518     MOVIW  0[FSR0] ; move distance into w

519     addlw  .10 ; 0x0A

520     MOVWI  0[FSR0]

521

522     ; If there was an overflow, add 1 to upper byte of current_value

523     MOVIW  1[FSR0]

524     btfsc  STATUS,C

525     addlw  .1

526     MOVWI  1[FSR0]

527

528     ; Check if result > MAX_DIST, if so set current value to MAX_DIST

529     call   add_error_check; error check: within limit?

530     btfsc  WREG,FAIL         ; if without limit, set to max value

531     call   set_max_value

532

533     goto   update_lcd ; write the new distance to the lcd

534

535 ; Jump to here from goto_menu when enter button pushed and "add 1" selected

536 ; Adds 1 to the value located at the address stored in current_value_pointer, then

537 ; error checks to make sure it never goes above the absolute maximum limit

538 add_1

539     movff16 current_value_pointer,FSR0

540

541     ; Add 1 to lower byte of current_value

542     MOVIW  0[FSR0] ; move distance into w

543     addlw  .1 ; 0x01

544     MOVWI  0[FSR0]

545

546     ; If there was an overflow, add 1 to upper byte of current_value

547     MOVIW  1[FSR0]

548     btfsc  STATUS,C

549     addlw  .1

550     MOVWI  1[FSR0]

551

552     ; Check if result > MAX_DIST, if so set current value to MAX_DIST

553     call   add_error_check; error check: within limit?

554     btfsc  WREG,FAIL         ; if without limit, set to max value

555     call   set_max_value

556

557     goto   update_lcd ; write the new distance to the lcd

558

559 ; Jump to here from goto_menu when enter button pushed and "sub 100" selected

560 ; Subtracts 100 from the value located at the address stored in

561 ; current_value_pointer, then error checks to make sure it never goes below

562 ; the absolute minimum limit

563 sub_100

564     movff16 current_value_pointer,FSR0

565     banksel temp

566

567     ; Move lower byte of current_value to temp, then subtract 100 from it and

568     ; move back into current_value

569     moviw   0[FSR0]

570     movwf   temp

571     movlw   .100 ; 0x64

572     subwf   temp,W

573     movwi   0[FSR0]

574

575     ; If lower byte underflowed, subtract 1 from upper byte of current_value

576     moviw   1[FSR0]

577     movwf   temp

578     movlw   .1

579     btfss  STATUS,C

580     subwf  temp,F

581     movf    temp,W

582     movwi   1[FSR0]

583

584     ; Check if result < MIN_DIST, if so set current value to MIN_DIST

585     call   sub_error_check ; error check: within limit?

586     btfsc  WREG,FAIL         ; if without limit, set to max value

587     call   set_min_value

588

589     goto   update_lcd ; write the new distance to the lcd

590

591 ; Jump to here from goto_menu when enter button pushed and "sub 10" selected

592 ; Subtracts 10 from the value located at the address stored in

593 ; current_value_pointer, then error checks to make sure it never goes below

594 ; the absolute minimum limit

595 sub_10

596     movff16 current_value_pointer,FSR0

597     banksel temp

598

599     ; Move lower byte of current_value to temp, then subtract 10 from it and

600     ; move back into current_value

601     moviw   0[FSR0]

602     movwf   temp

603     movlw   .10 ; 0x64

604     subwf   temp,W

605     movwi   0[FSR0]

606

607     ; If lower byte underflowed, subtract 1 from upper byte of current_value

608     moviw   1[FSR0]

609     movwf   temp

610     movlw   .1

611     btfss  STATUS,C

612     subwf  temp,F

613     movf    temp,W

614     movwi   1[FSR0]

615

616     ; Check if result < MIN_DIST, if so set current value to MIN_DIST

617     call   sub_error_check ; error check: within limit?

618     btfsc  WREG,FAIL         ; if without limit, set to max value

619     call   set_min_value

620

621     goto   update_lcd ; write the new distance to the lcd

622

623 ; Jump to here from goto_menu when enter button pushed and "sub 1" selected

624 ; Subtracts 1 from the value located at the address stored in

625 ; current_value_pointer, then error checks to make sure it never goes below

626 ; the absolute minimum limit

627 sub_1

628     movff16 current_value_pointer,FSR0

629     banksel temp

630

631     ; Move lower byte of current_value to temp, then subtract 1 from it and

632     ; move back into current_value

633     moviw   0[FSR0]

634     movwf   temp

635     movlw   .1 ; 0x64

636     subwf   temp,W

637     movwi   0[FSR0]

638

639     ; If lower byte underflowed, subtract 1 from upper byte of current_value

640     moviw   1[FSR0]

641     movwf   temp

642     movlw   .1

643     btfss  STATUS,C

644     subwf  temp,F

645     movf    temp,W

646     movwi   1[FSR0]

647

648     call   sub_error_check ; error check: within limit?

649     btfsc  WREG,FAIL         ; if without limit, set to max value

650     call   set_min_value

651

652     ; Check if result < MIN_DIST, if so set current value to MIN_DIST

653     goto   update_lcd ; write the new distance to the lcd

654

655 set_max_value

656     movli16 DIST_MAX,FSR0

657     return

658

659 set_min_value

660     movli16 DIST_MIN,FSR0

661     return

662

663 ; See if current_value > DIST_MAX by seeing if (current_value - DIST_MAX)

664 ; underflows and sets the carry bit

665 add_error_check

666     MOVIW   0[FSR0]

667     movwf   X

668     MOVIW   1[FSR0]

669     movwf   X+1

670

671     movlf16 DIST_MAX,Y

672

673     call    subt16x16

674

675     btfss   STATUS,C

676     goto    add_error_check_fail

677

678     goto    add_error_check_pass

679 add_error_check_fail

680     retlw FAIL

681 add_error_check_pass

682     retlw PASS

683

684 ; See if current_value < DIST_MIN by seeing if (DIST_MIN - current_value)

685 ; underflows and sets the carry bit

686 sub_error_check

687     MOVIW 0[FSR0]

688     movwf Y

689     MOVIW 1[FSR0]

690     movwf Y+1

691

692     movlf16 DIST_MIN,X

693

694     call  subt16x16

695

696     btfss STATUS,C

697     goto sub_error_check_fail

698

699     goto sub_error_check_pass

700 sub_error_check_fail

701     retlw FAIL

702 sub_error_check_pass

703     retlw PASS

704

705 ; Call scan, get result (ie passed, within tolerance, failed, or timeout

706 ; Then choose goto to print appropriate string

707 perform_scan

708     call    scan

709

710     btfsc   usresult,PASS

711     goto    print_pass_string

712

713     btfsc   usresult,TOLERANCE

714     goto    print_tolerance_string

715

716     btfsc   usresult,FAIL

717     goto    print_fail_string

718

719     btfsc   usresult,TIMEOUT

720     goto    print_timeout_string

721

722 ; Prepare for Start Menu to become current menu

723 set_start

724     ; Reset menu_selected

725     movlf   START_MENU_MIN,menu_selected

726

727     ; Set minimum possible menu

728     movlf   START_MENU_MIN,menu_min

729

730     ; Set maximum possible menu

731     movlf   START_MENU_MAX,menu_max

732

733     ; Copy location of start menu to menu_pointer

734     movlf16 start_menu,menu_pointer

735

736     ; Copy location of info for printing lcd for the start meny

737     movlf16 update_lcd_start_menu,lcd_pointer

738

739     goto  update_lcd

740

741 ; Prepare for Upper Tolerance Menu to become current menu

742 set_upper_tolerance

743     movlf   UPPER_TOLERANCE_MENU_MIN,menu_selected

744     movlf   UPPER_TOLERANCE_MENU_MIN,menu_min

745     movlf   UPPER_TOLERANCE_MENU_MAX,menu_max

746     movlf16 upper_tolerance,current_value_pointer

747     movlf16 upper_tolerance_menu,menu_pointer

748     movlf16 update_lcd_upper_tolerance_menu,lcd_pointer

749     goto  update_lcd

750

751 ; Prepare for Lower Tolerance Menu to become current menu

752 set_lower_tolerance

753     movlf   LOWER_TOLERANCE_MENU_MIN,menu_selected

754     movlf   LOWER_TOLERANCE_MENU_MIN,menu_min

755     movlf   LOWER_TOLERANCE_MENU_MAX,menu_max

756     movlf   lower_tolerance,current_value_pointer

757     movlf16 lower_tolerance_menu,menu_pointer

758     movlf16 update_lcd_lower_tolerance_menu,lcd_pointer

759     goto  update_lcd

760

761 ; Prepare for Upper Passband Menu to become current menu

762 set_upper_passband

763     movlf   UPPER_PASSBAND_MENU_MIN,menu_selected

764     movlf   UPPER_PASSBAND_MENU_MIN,menu_min

765     movlf   UPPER_PASSBAND_MENU_MAX,menu_max

766     movlf16   upper_passband,current_value_pointer

767     movlf16 upper_passband_menu,menu_pointer

768     movlf16 update_lcd_upper_passband_menu,lcd_pointer

769     goto  update_lcd

770

771 ; Prepare for Lower Passband to become current menu

772 set_lower_passband

773     movlf   LOWER_PASSBAND_MENU_MIN,menu_selected

774     movlf   LOWER_PASSBAND_MENU_MIN,menu_min

775     movlf   LOWER_PASSBAND_MENU_MAX,menu_max

776     movlf16   lower_passband,current_value_pointer

777     movlf16 lower_passband_menu,menu_pointer

778     movlf16 update_lcd_lower_passband_menu,lcd_pointer

779     goto  update_lcd

780

781 ; Jump to the appropriate print function for Start Menu

782 update_lcd_start_menu

783     movf menu_selected,W

784     brw

785     NOP ;error

786     NOP ;error

787     goto print_scan_string

788     goto print_upper_passband_string

789     goto print_lower_passband_string

790     goto print_upper_tolerance_string

791     goto print_lower_tolerance_string

792

793 ; Jump to the appropriate print function for Upper Passband Menu

794 update_lcd_upper_passband_menu

795     movf menu_selected,W

796     brw

797     NOP ;error

798     NOP ;error

799     goto print_add_100_string

800     goto print_add_10_string

801     goto print_add_1_string

802     goto print_sub_100_string

803     goto print_sub_10_string

804     goto print_sub_1_string

805     goto print_start_string

806

807 ; Jump to the appropriate print function for Lower Passband Menu

808 update_lcd_lower_passband_menu

809     movf menu_selected,W

810     brw

811     NOP ;error

812     NOP ;error

813     goto print_add_100_string

814     goto print_add_10_string

815     goto print_add_1_string

816     goto print_sub_100_string

817     goto print_sub_10_string

818     goto print_sub_1_string

819     goto print_start_string

820

821 ; Jump to the appropriate print function for Upper Tolerance Menu

822 update_lcd_upper_tolerance_menu

823     movf menu_selected,W

824     brw

825     NOP ;error

826     NOP ;error

827     goto print_add_100_string

828     goto print_add_10_string

829     goto print_add_1_string

830     goto print_sub_100_string

831     goto print_sub_10_string

832     goto print_sub_1_string

833     goto print_start_string

834

835 ; Jump to the appropriate print function for Lower Tolerance Menu

836 update_lcd_lower_tolerance_menu

837     movf menu_selected,W

838     brw

839     NOP ;error

840     NOP ;error

841     goto print_add_100_string

842     goto print_add_10_string

843     goto print_add_1_string

844     goto print_sub_100_string

845     goto print_sub_10_string

846     goto print_sub_1_string

847     goto print_start_string

848

849 ; Print "START" to top line of the LCD

850 print_start_string

851     call    lcd_clear ; clear the lcd

852     call    set_lcd_top_line ; set lcd to write on top line

853     movlf16 start_string,FSR0

854     call    lcd_print      ; write scan_string to lcd_print

855     goto    fin

856

857 ; Print "SCAN" to top line of the LCD

858 print_scan_string

859     call    lcd_clear ; clear the lcd

860     call    set_lcd_top_line ; set lcd to write on top line

861     movlf16 scan_string,FSR0

862     call    lcd_print      ; write scan_string to lcd_print

863     goto    fin

864

865 ; Print "SCAN" to top line of the LCD, and "PASS: XXXX CM" to bottom line of lcd

866 ; where XXXX is the four-digit result from scan. Sets GREEN_LED, and clears

867 ; AMBER_LED and RED_LED

868 print_pass_string

869     call    hex16_2_ascii

870

871     call    lcd_clear

872     call    set_lcd_top_line

873     movlf16 scan_string, FSR0

874     call    lcd_print

875

876     call    set_lcd_bottom_line

877     movlf16 pass_string, FSR0

878     call    lcd_print

879     movlf16 ascii, FSR0

880     call    lcd_print

881     movlf16 mm_string, FSR0

882     call    lcd_print

883

884     banksel LEDPORT

885     bsf     LEDPORT, GREEN_LED

886     bcf     LEDPORT, AMBER_LED

887     bcf     LEDPORT, RED_LED

888

889     clear usresult

890

891     goto    fin

892

893 ; Print "SCAN" to top line of the LCD, and "FAIL: XXXX CM" to bottom line of lcd

894 ; where XXXX is the four-digit result from scan. Sets AMBER_LED, and clears

895 ; GREEN_LED and RED_LED

896 print_tolerance_string

897     call    hex16_2_ascii

898

899     call  lcd_clear ; clear the lcd

900     call  set_lcd_top_line ; set lcd to write on top line

901     movlf16 scan_string, FSR0

902     call    lcd_print

903

904     call    set_lcd_bottom_line

905     movlf16 fail_string, FSR0

906     call    lcd_print

907     movlf16 ascii, FSR0

908     call    lcd_print

909     movlf16 mm_string, FSR0

910     call    lcd_print

911

912     banksel LEDPORT

913     bcf     LEDPORT, GREEN_LED

914     bsf     LEDPORT, AMBER_LED

915     bcf     LEDPORT, RED_LED

916

917     clear usresult

918

919     goto    fin

920

921 ; Print "SCAN" to top line of the LCD, and "FAIL: XXXX CM" to bottom line of lcd

922 ; where XXXX is the four-digit result from scan. Sets RED_LED, and clears

923 ; GREEN_LED and AMBER_LED

924 print_fail_string

925     ; value should already be in X

926     call    hex16_2_ascii

927

928     call  lcd_clear ; clear the lcd

929     call  set_lcd_top_line ; set lcd to write on top line

930     movlf16 scan_string, FSR0

931     call    lcd_print

932

933     call    set_lcd_bottom_line

934     movlf16 fail_string, FSR0

935     call    lcd_print

936     movlf16 ascii, FSR0

937     call    lcd_print

938     movlf16 mm_string, FSR0

939     call    lcd_print

940

941     banksel LEDPORT

942     bcf     LEDPORT, GREEN_LED

943     bcf     LEDPORT, AMBER_LED

944     bsf     LEDPORT, RED_LED

945

946     clear usresult

947

948     goto    fin

949

950 ; Print "SCAN" to top line of the LCD, and "TIMEOUT" to bottom line of lcd

951 ; Clears all three LEDs

952 print_timeout_string

953     call    lcd_clear ; clear the lcd

954     call    set_lcd_top_line ; set lcd to write on top line

955     movlf16 scan_string, FSR0

956     call    lcd_print

957

958     call    set_lcd_bottom_line

959     movlf16 timeout_string, FSR0

960     call    lcd_print

961

962     banksel LEDPORT

963     bCf     LEDPORT, GREEN_LED

964     bcf     LEDPORT, AMBER_LED

965     bsf     LEDPORT, RED_LED

966

967     bcf LEDPORT, GREEN_LED

968     bcf LEDPORT, AMBER_LED

969     bcf LEDPORT, RED_LED

970

971     clear usresult

972

973     goto    fin

974

975 ; Print "UPPER PASSBAND" to top line of the LCD

976 print_upper_passband_string

977     call  lcd_clear

978     call  set_lcd_top_line

979     movlf   upper_passband_string,FSR0

980     call  lcd_print

981     goto    fin

982

983 ; Print "LOWER PASSBAND" to top line of the LCD

984 print_lower_passband_string

985     call  lcd_clear

986     call  set_lcd_top_line

987     movlf16   lower_passband_string,FSR0

988     call  lcd_print

989     goto  fin

990

991 ; Print "UPPER TOLERANCE" to top line of the LCD

992 print_upper_tolerance_string

993     call  lcd_clear

994     call  set_lcd_top_line

995     movlf16  upper_tolerance_string,FSR0

996     call  lcd_print

997     goto  fin

998

999 ; Print "LOWER TOLERANCE" to top line of the LCD

1000 print_lower_tolerance_string

1001     call  lcd_clear

1002     call  set_lcd_top_line

1003     movlf16   lower_tolerance_string,FSR0

1004     call  lcd_print

1005     goto  fin

1006

1007 ; Print "ADD 100" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1008 ; where XXXX is the four-digit value of (current_value + 100)

1009 print_add_100_string

1010     movff16 current_value_pointer,FSR0

1011     moviw16 FSR0,X

1012     call    hex16_2_ascii

1013

1014     call    lcd_clear

1015     call    set_lcd_top_line

1016     movlf16 add_100_string,FSR0

1017     call    lcd_print

1018

1019     call    set_lcd_bottom_line

1020     movlf   ascii,FSR0

1021     bcf     FSR0H,7

1022     call    lcd_print

1023     movlf   mm_string,FSR0

1024     call    lcd_print

1025

1026     goto    fin

1027

1028 ; Print "ADD 10" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1029 ; where XXXX is the four-digit resut of (current_value + 10)

1030 print_add_10_string

1031     movff16 current_value_pointer,FSR0

1032     moviw16 FSR0,X

1033     call    hex16_2_ascii

1034

1035     call    lcd_clear

1036     call    set_lcd_top_line

1037     movlf16 add_10_string,FSR0

1038     call    lcd_print

1039

1040     call    set_lcd_bottom_line

1041     movlf   ascii,FSR0

1042     ; FSR0H.7 indicates location is in program memory, clear it just in case

1043     ; it hasn't been cleared, since ascii is in data memory

1044     bcf     FSR0H,7

1045     call    lcd_print

1046     movlf   mm_string,FSR0

1047     call    lcd_print

1048

1049     goto    fin

1050

1051 ; Print "ADD 1" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1052 ; where XXXX is the four-digit value of (current_value + 1)

1053 print_add_1_string

1054     movff16 current_value_pointer,FSR0

1055     moviw16 FSR0,X

1056     call    hex16_2_ascii

1057

1058     call    lcd_clear

1059     call    set_lcd_top_line

1060     movlf16 add_1_string,FSR0

1061     call    lcd_print

1062

1063     call    set_lcd_bottom_line

1064     movlf   ascii,FSR0

1065     ; FSR0H.7 indicates location is in program memory, clear it just in case

1066     ; it hasn't been cleared, since ascii is in data memory

1067     bcf     FSR0H,7

1068     call    lcd_print

1069     movlf   mm_string,FSR0

1070     call    lcd_print

1071

1072     goto    fin

1073

1074 ; Print "SUB 100" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1075 ; where XXXX is the four-digit value of (current_value - 1)

1076 print_sub_100_string

1077     movff16 current_value_pointer,FSR0

1078     moviw16 FSR0,X

1079     call    hex16_2_ascii

1080

1081     call    lcd_clear

1082     call    set_lcd_top_line

1083     movlf16 sub_100_string,FSR0

1084     call    lcd_print

1085

1086     call    set_lcd_bottom_line

1087     movlf   ascii,FSR0

1088     ; FSR0H.7 indicates location is in program memory, clear it just in case

1089     ; it hasn't been cleared, since ascii is in data memory

1090     bcf     FSR0H,7

1091     call    lcd_print

1092     movlf   mm_string,FSR0

1093     call    lcd_print

1094

1095     goto    fin

1096

1097 ; Print "SUB 10" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1098 ; where XXXX is the four-digit value of (current_value - 10)

1099 print_sub_10_string

1100     movff16 current_value_pointer,FSR0

1101     moviw16 FSR0,X

1102     call    hex16_2_ascii

1103

1104     call    lcd_clear

1105     call    set_lcd_top_line

1106     movlf16 sub_10_string,FSR0

1107     call    lcd_print

1108

1109     call    set_lcd_bottom_line

1110     movlf   ascii,FSR0

1111     ; FSR0H.7 indicates location is in program memory, clear it just in case

1112     ; it hasn't been cleared, since ascii is in data memory

1113     bcf     FSR0H,7

1114     call    lcd_print

1115     movlf   mm_string,FSR0

1116     call    lcd_print

1117

1118     goto    fin

1119

1120 ; Print "SUB 1" to top line of the LCD, and "XXXX MM" to bottom line of lcd

1121 ; where XXXX is the four-digit value of (current_value - 1)

1122 print_sub_1_string

1123     movff16 current_value_pointer,FSR0

1124     moviw16 FSR0,X

1125     call    hex16_2_ascii

1126

1127     call    lcd_clear

1128     call    set_lcd_top_line

1129     movlf16 sub_1_string,FSR0

1130     call    lcd_print

1131

1132     call    set_lcd_bottom_line

1133     movlf   ascii,FSR0

1134     ; FSR0H.7 indicates location is in program memory, clear it just in case

1135     ; it hasn't been cleared, since ascii is in data memory

1136     bcf     FSR0H,7

1137     call    lcd_print

1138     movlf   mm_string,FSR0

1139     call    lcd_print

1140

1141     goto    fin

1142

1143 ;*******************************************************************************

1144 ; SCAN

1145 ;

1146 ; Gets a result from the Polaroid 6500, in units of microseconds

1147 ; Processes it, and then returns a value indicating whether passed, failed,

1148 ; within tolerance or timed out

1149 ;

1150 ; Starts with a 16-bit result from the Polaroid 6500 in microseconds

1151 ;

1152 ; The result is processed as follows:

1153 ;

1154 ; 1. (((result from Polaroid 6500) - PULSEGEN) * MM_PER_US)/1024) - LATENCY

1155 ; 2. The above is then compared to passband, if within the condition is PASS

1156 ; 3. Then compared to tolerance, if within condition is TOLERANCE

1157 ; 4. Must be outside passband or tolerance, therefore condition is FAIL

1158 ;

1159 ; If the PIC doesn't detect a reponse from ECHO before TMR1 overflow, then

1160 ; condition is TIMEOUT

1161 ;*******************************************************************************

1162 scan

1163                     call    ultrasound

1164

1165                     banksel usresult

1166                     btfsc   usresult,PASS

1167                     goto    scan_pass

1168                     btfsc   usresult,FAIL

1169                     goto    scan_timeout

1170 scan_pass           clear   usresult

1171                     ; Recover result from CCPR1

1172                     movff   CCPR1L,X

1173                     movff   CCPR1H,X+1

1174                     ;movlf16   0x1D60,X

1175                     ; subtract ultrasound clock generation delay

1176                     movlf16 PULSEGEN,Y

1177                     call    subt16x16

1178                     ; multiply by (mm per instruction clock cycle)*1024

1179                     movff   result,X

1180                     movff   result+1,X+1

1181                     movlf16 MM_PER_US,Y

1182                     call    mult16x16       ;

1183                     ; divide by 1024 to get (mm per instruction clock cycle)

1184                     movlf   .10,Y

1185                     call    div40bin

1186                     ; subtract 20cm from result, fiddle factor

1187                     movff16 result,X

1188                     movlf16 LATENCY,Y

1189                     call    subt16x16

1190                     ; check if the result is less than the lower limit

1191                     movff16 result,X ; expect a 16 bit result so this is ok

1192                     movff16 lower_passband,Y

1193                     call    subt16x16

1194                     btfss   STATUS,C        ;|\ If Z set, then either X = Y, or

1195                     goto    below_passband     ;// X > Y If Z clear, then X < Y

1196                     movwf   temp            ;

1197                     ; check if result is greater than the upper limit

1198                     movff16 upper_passband,Y

1199                     call    subt16x16

1200                     btfsc   STATUS,C

1201                     goto    above_passband

1202                     ; if you get here then hooray, the value is in the passband

1203                     bsf     usresult,PASS

1204                     goto    scan_end

1205

1206 below_passband

1207                     ; test if within tolerance

1208                     movff16 lower_tolerance,Y

1209                     call    subt16x16

1210                     btfss   STATUS,C

1211                     goto    scan_fail

1212                     ; if you get here then the value is within tolerance

1213                     bsf     usresult, TOLERANCE

1214                     goto    scan_end

1215

1216 above_passband

1217                     movff16 upper_tolerance,Y

1218                     call    subt16x16

1219                     btfsc   STATUS,C

1220                     goto    scan_fail

1221                     ; test if within tolerance

1222                     ; if you get here then within tolerance

1223                     bsf     usresult, TOLERANCE

1224                     goto scan_end

1225

1226 scan_fail

1227                     bsf     usresult, FAIL

1228                     goto    scan_end

1229 scan_timeout

1230                     clear   usresult

1231                     bsf     usresult,TIMEOUT

1232                     goto    scan_end

1233 scan_end

1234                     return

1235

1236 ; Communicate with Polaroid 6500 and get a result in terms of time

1237 ultrasound

1238                 ; Clear GIE because we don't want to use the ISR. Enable PEIE so

1239                 ; that Interrupt Flags can operate

1240                 clrf    INTCON

1241                 bcf     INTCON,GIE

1242                 bsf     INTCON,PEIE

1243                 ; Set TMR1IE and CCP1IE because we will use these modules

1244                 banksel PIE1

1245                 bsf     PIE1,TMR1IE

1246                 bsf     PIE1,CCP1IE

1247                 ; Use instruction clock as Timer 1 input, 1MHz -> 1us per

1248                 ; increment

1249                 movlf   0x05,T1CON

1250                 ; Set INIT to begin scan

1251                 banksel PORTB

1252                 bsf     PORTB,INIT

1253                 ; Clean TMR1 out just in case it isn't empty

1254                 banksel TMR1H

1255                 clrf    TMR1H

1256                 clrf    TMR1L

1257                 bcf     INTCON,TMR0IF

1258                 ; Reset TMR0 configuration

1259                 banksel OPTION_REG

1260                 bsf     OPTION_REG,TMR0CS

1261                 movlf   0x05,CCP1CON

1262                 ; Poll IFs in infinite loop until we have a result or timeout

1263                 banksel PIR1

1264 wait_loop

1265                 ; If TMR1IF set, then overflow ergo ultrasound must not be

1266                 ; responding

1267                 btfsc   PIR1,TMR1IF

1268                 goto    timeout

1269                 ; If CCP1IF set, then we must have got a result from the

1270                 ; Polaroid 6500

1271                 btfsc   PIR1,CCP1IF

1272                 goto    success

1273                 ; If neither IF's are set, loop back until one is

1274                 goto    wait_loop

1275 timeout

1276                 banksel usresult

1277                 bsf     usresult,FAIL

1278                 goto    got_result

1279 success

1280                 banksel usresult

1281                 bsf     usresult,PASS

1282                 goto    got_result

1283

1284 got_result

1285                 ; Clear INIT to stop the Polaroid 6500 repeating the test

1286                 banksel PORTB

1287                 bcf     PORTB,INIT

1288                 ; Switch peripheral interrupts off because we don't need them

1289                 ; right now

1290                 bcf     INTCON,PEIE

1291                 ; Disable CCP1, TMR1, and PIR1

1292                 clear   CCP1CON

1293                 clear   T1CON

1294                 clear   PIR1

1295                 ; Clear Interrupt Flags

1296                 clear   PIE1

1297                 ; Set TMR0 to overflow after 80ms elapsed

1298                 movlf   0x12,OSCCON

1299                 bcf     INTCON,TMR0IF

1300                 movlf   0x62,TMR0

1301                 ; Set TMR0 up with a prescaler of 4 and to use instruction clock

1302                 ; (1Mhz)

1303                 movlf   0x01,OPTION_REG

1304 ults_end

1305                 ; Wait until 80ms elapsed before moving on

1306                 btfss   INTCON,TMR0IF

1307                 goto    $-1

1308                 ; Disable TMR0

1309                 bcf     INTCON,TMR0IF

1310                 ; Reset TMR0 configuration

1311                 banksel OPTION_REG

1312                 bsf     OPTION_REG,TMR0CS

1313                 ; Reset System Clock to 4MHz

1314                 movlf   0xEB,OSCCON

1315                 return

1316

1317 ;*******************************************************************************

1318 ; MATHS FUNCTIONS

1319 ;

1320 ;******************************************************************************

1321

1322 ; Multiply together 16-bit integer X, and 16-bit integer Y and place into 40-bit

1323 ; Result register

1324 mulxy   MACRO   byte,bit

1325         btfsc   byte,bit

1326         call    add_y

1327         rrf result+4,F

1328         rrf result+3,F

1329         rrf result+2,F

1330         rrf result+1,F

1331         rrf result,F

1332         ENDM

1333 mult16x16

1334         bcf     STATUS,C  ; Clear Carry bit before we use it

1335         banksel X

1336         clrf    result+4   ; \  Clear registers that hold result

1337         clrf    result+3   ;  |

1338         clrf    result+2   ;  |

1339         clrf    result+1   ;  |

1340         clrf    result     ; /

1341         mulxy   X,0   ; \  Perform long multiplication on X and Y, make the

1342         mulxy   X,1   ;  |

1343         mulxy   X,2   ;  |

1344         mulxy   X,3   ;  |

1345         mulxy   X,4   ;  |

1346         mulxy   X,5   ;  |

1347         mulxy   X,6   ;  |

1348         mulxy   X,7   ;  |

1349         mulxy   X+1,0 ;  |

1350         mulxy   X+1,1 ;  |

1351         mulxy   X+1,2 ;  |

1352         mulxy   X+1,3 ;  |

1353         mulxy   X+1,4 ;  |

1354         mulxy   X+1,5 ;  |

1355         mulxy   X+1,6 ;  |

1356         mulxy   X+1,7 ; /

1357         return

1358 add_y

1359         banksel Y

1360         movf    Y,W          ;\ Add least significant byte of Y to 2nd most

1361         addwf   result+2,F   ;/ sig. byte of result, if overflow set carry bit

1362         movf    Y+1,W        ;\ Add most significant byte of Y and C to most

1363         addwfc  result+3,F   ;/ sig. byte of result, if overflow set carry bit

1364         btfss   STATUS,C     ;\ Carry bit set -> result+3 oflows into result+4

1365         goto    fin_y        ;| .'. add carry bit to result+4

1366         movlw   0x00         ;|

1367         addwfc  result+4     ;/

1368 fin_y   return

1369

1370 ; Add 16-bit integer Y to 16-bit integer X, and place into result register

1371 add16x16

1372         movf   Y,W         ;\ Add lower byte of Y to lower byte of X

1373         addwf  X,W         ;|

1374         movwf  result      ;/

1375         movf   Y+1,W       ;\ Add upper byte of Y to upper byte of X

1376         addwfc X+1,W       ;|

1377         movwf  result+1    ;/

1378         movwf  0x00        ;\ Add carry to overflow

1379         addwfc result+2,F  ;/

1380         return       ;

1381

1382 ; Subtract a 16-bit integer Y from 16-bit integer X, place into result register

1383 subt16x16

1384             banksel Y

1385             movf    Y,W

1386             subwf   X,W

1387             movwf   result

1388             ;

1389             movf    Y+1,W

1390             btfss   STATUS,C

1391             addlw   0x01

1392             subwf   X+1,W

1393             movwf   result+1

1394             ;

1395             return

1396

1397 ; Divide 40-bit wide result register by 1024 by bit shifting 10 places

1398 div40bin

1399 div         bcf  STATUS,C    ;|

1400             rrf  result+4,f  ;|\ Shift right from MSreg to LSreg

1401             rrf  result+3,f  ;||

1402             rrf  result+2,f  ;||

1403             rrf  result+1,f  ;||

1404             rrf  result,f    ;|/

1405             decfsz  Y,f      ;| Decrement count, exit loop when zero

1406             goto    div      ;/

1407             return

1408

1409 ;*******************************************************************************

1410 ; LCD FUNCTIONS

1411 ;*******************************************************************************

1412

1413 lcd_setup

1414             clear   ANSELA

1415             clear   LCDDIR

1416             clear   LCDPORT

1417             ; Set WDT for 1s

1418             movlf   0x15,WDTCON

1419

1420             sleep

1421

1422             clear   WDTCON

1423

1424             movlf   .10,deltime

1425

1426             movlw       0x02         ;Set 4 bit mode

1427             call        enable

1428             call        delay

1429

1430             movlw       0x02         ;Set 4 bit mode

1431             call        enable

1432             call        delay

1433

1434             movlw       0x08         ;Set 2 line

1435             call        enable

1436             call        delay

1437

1438             movlf       0x0F, cmdbyte ;Display on, cursor on, blinking on

1439             call        lcdcmd

1440

1441             movlf       0x06, cmdbyte ;Entry mode set.

1442             call        lcdcmd

1443

1444             return

1445

1446 lcdbusy

1447             banksel LCDDIR

1448             bsf     LCDDIR,DB7

1449             clear   LCDPORT

1450             bsf     LCDPORT,RW

1451

1452 lcdbsy1     bsf     LCDPORT,EN

1453             nop

1454             movf    LCDPORT,w

1455             bcf     LCDPORT,EN

1456             nop

1457

1458             bsf     LCDPORT,EN

1459             nop

1460             bcf     LCDPORT,EN

1461             nop

1462

1463             btfsc   WREG,DB7

1464             goto    lcdbsy1

1465

1466             clear   LCDDIR

1467             return

1468

1469 lcdcmd

1470             swapf       cmdbyte,w   ;\

1471             andlw       LCDMASK     ; | Output the LCD command stored in cmdbyte

1472             call        enable      ; |

1473

1474             movf        cmdbyte,w   ; | Outputs high nibble then low nibble.

1475             andlw       LCDMASK     ; |

1476             call        enable      ; |

1477                                     ; |

1478             call        lcdbusy     ; |

1479                                     ; |

1480             return                  ;/

1481

1482 set_lcd_top_line

1483             movlf       0x80,cmdbyte ; Move cursor to line 1

1484             call        lcdcmd

1485             return

1486

1487 ; Move cursor to line 2

1488 set_lcd_bottom_line

1489

1490             movlf       0xC0, cmdbyte

1491             call        lcdcmd

1492             return

1493

1494 ; Clear screen

1495 lcd_clear

1496             movlf       0x01, cmdbyte

1497             call        lcdcmd

1498             return

1499

1500 ; Turn screen off

1501 lcdoff

1502             movlf       0x08, cmdbyte

1503             call        lcdcmd

1504             return

1505

1506 ; Turn Screen on

1507 lcdon

1508             movlf       0x0C, cmdbyte

1509             call        lcdcmd

1510             return

1511

1512 ; Print string to LCD

1513 lcd_print

1514             moviw       0[FSR0]

1515             andlw       0xFF

1516             btfsc       STATUS,Z    ;

1517             return                  ;

1518             swapf       WREG,w

1519             andlw       LCDMASK     ; Move first string byte from FSR, if zero

1520             bsf         WREG,RS     ; (null), return as end of string reached.

1521             call        enable

1522

1523             moviw       FSR0++      ; Send high then low nibble of character

1524             andlw       LCDMASK     ; incrementing FSR then call 'lcdbusy'.

1525             bsf         WREG,RS

1526             call        enable

1527

1528             call        lcdbusy

1529

1530             goto        lcd_print    ; Will loop until end of string reached.

1531

1532 enable

1533             banksel     LCDPORT     ;\

1534             movwf       LCDPORT     ; | Loads nibble onto LCD data lines, sets

1535             bsf         LCDPORT,EN  ; | the enable line then clears the LCD

1536             nop                     ; | port.

1537             clrf        LCDPORT     ; |

1538             return                  ;/

1539

1540 delay       movf        deltime,w   ;\

1541             goto        deljmp      ; | Should be a 10us delay each loop, uses

1542 delloop     nop                     ; | number stored in 'deltime' for iteration

1543             nop                     ; | count.

1544             nop                     ; |

1545             nop                     ; | First loop shorter due to instruction

1546             nop                     ; | times calling function.

1547             nop                     ; |

1548 deljmp      nop                     ; | 10us only valid for 4MHz clock.

1549             decfsz      WREG,w      ; |

1550             goto        delloop     ; |

1551             return                  ;/

1552

1553 ;*******************************************************************************

1554 ; HEX16 TO ASCII

1555 ;

1556 ; Convert a 16-bit register X:2 into a 4 radix wide denary number, ascii:5

1557 ;*******************************************************************************

1558 hex16_2_ascii

1559             call hex16_2_bcd

1560             get_ascii_un    BCD+1, ascii

1561             get_ascii_ln    BCD+1, ascii+1

1562             get_ascii_un    BCD+2, ascii+2

1563             get_ascii_ln    BCD+2, ascii+3

1564             movlf           "\0",  ascii+4

1565

1566             return

1567

1568 hex16_2_bcd

1569     bcf STATUS, C

1570     movlf   .16, count

1571     clear   BCD

1572     clear   BCD+1

1573     clear   BCD+2

1574     clear   BCD+3

1575 hex16_2_bcd_loop

1576     banksel X

1577     rlf X,f

1578     rlf X+1,f

1579

1580     banksel BCD

1581     rlf BCD+2,f

1582     rlf BCD+1,f

1583     rlf BCD,f

1584

1585     banksel count

1586     decfsz  count,f

1587     goto    adjDEC

1588

1589     return

1590

1591 adjDEC

1592     movlf   BCD+2,FSR1

1593     call    adjBCD

1594

1595     movlf   BCD+1,FSR1

1596     call    adjBCD

1597

1598     movlf   BCD,FSR1

1599     call    adjBCD

1600

1601     goto    hex16_2_bcd_loop

1602

1603 adjBCD

1604     movf    INDF1,w

1605     addlw   0x03

1606     btfsc   WREG,3

1607     movwi   0[FSR1]

1608

1609     moviw   0[FSR1]

1610     addlw   0x30

1611     btfsc   WREG,7

1612     movwi   0[FSR1]

1613

1614     return

1615

1616 ; get_ascii

1617 ; Returns the ASCII equivalent of a single BCD number

1618 ; Uses the least significant nibble

1619 get_ascii

1620     brw

1621     retlw   "0"

1622     retlw   "1"

1623     retlw   "2"

1624     retlw   "3"

1625     retlw   "4"

1626     retlw   "5"

1627     retlw   "6"

1628     retlw   "7"

1629     retlw   "8"

1630     retlw   "9"

1631

1632 ;*******************************************************************************

1633 ; SOFTWARE ERROR

1634 ; You don't want to end up here

1635 ; Sets all three LEDs ON

1636 ;*******************************************************************************

1637

1638 software_error

1639     BANKSEL TRISB

1640     bsf     TRISB,GREEN_LED

1641     bsf     TRISB,AMBER_LED

1642     bsf     TRISB,RED_LED

1643     goto    $

1644

1645                 end

1646

# Appendix C: .map File

MPLINK 5.01, LINKER

Linker Map File - Created Wed Jan 28 23:12:04 2015

Section Info

Section       Type    Address   Location Size(Bytes)

---------  ---------  ---------  ---------  ---------

.org_0       code   0x000000    program   0x000002

.cinit    romdata   0x000001    program   0x000004

.org_1       code   0x000004    program   0x000c7a

Program Memory Usage

Start         End

---------   ---------

0x000000    0x000002

0x000004    0x000640

1600 out of 8455 program addresses used, program memory utilization is 18%

Symbols - Sorted by Name

Name    Address   Location    Storage File

---------  ---------  ---------  --------- ---------

ISR   0x000004    program     static D:\en0627_assignment.X\en0627_assignment.asm

TMR0_reset   0x0000df    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_00B7   0x0000b7    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_04CC   0x0004cc    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_0640   0x000640    program     static D:\en0627_assignment.X\en0627_assignment.asm

above_passband   0x000482    program     static D:\en0627_assignment.X\en0627_assignment.asm

add16x16   0x000556    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_1   0x000168    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_10   0x000155    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_100   0x000142    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_100_string   0x000069    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_10_string   0x000071    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_1_string   0x000078    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check   0x0001d0    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check_fail   0x0001dd    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check_pass   0x0001de    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_y   0x00054c    program     static D:\en0627_assignment.X\en0627_assignment.asm

adjBCD   0x000628    program     static D:\en0627_assignment.X\en0627_assignment.asm

adjDEC   0x00061b    program     static D:\en0627_assignment.X\en0627_assignment.asm

below_passband   0x000475    program     static D:\en0627_assignment.X\en0627_assignment.asm

cycle_left   0x0000eb    program     static D:\en0627_assignment.X\en0627_assignment.asm

cycle_right   0x0000ef    program     static D:\en0627_assignment.X\en0627_assignment.asm

delay   0x0005db    program     static D:\en0627_assignment.X\en0627_assignment.asm

deljmp   0x0005e3    program     static D:\en0627_assignment.X\en0627_assignment.asm

delloop   0x0005dd    program     static D:\en0627_assignment.X\en0627_assignment.asm

div   0x000569    program     static D:\en0627_assignment.X\en0627_assignment.asm

div40bin   0x000569    program     static D:\en0627_assignment.X\en0627_assignment.asm

enable   0x0005d5    program     static D:\en0627_assignment.X\en0627_assignment.asm

error_check_lower   0x0000fe    program     static D:\en0627_assignment.X\en0627_assignment.asm

error_check_upper   0x000105    program     static D:\en0627_assignment.X\en0627_assignment.asm

fail_string   0x00004f    program     static D:\en0627_assignment.X\en0627_assignment.asm

fin   0x0000fb    program     static D:\en0627_assignment.X\en0627_assignment.asm

fin_y   0x000555    program     static D:\en0627_assignment.X\en0627_assignment.asm

get_ascii   0x000631    program     static D:\en0627_assignment.X\en0627_assignment.asm

got_result   0x0004b6    program     static D:\en0627_assignment.X\en0627_assignment.asm

goto_menu   0x0000e1    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_ascii   0x0005e7    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_bcd   0x000604    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_bcd_loop   0x000610    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_clear   0x0005b8    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_print   0x0005c7    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_setup   0x000572    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdbsy1   0x000598    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdbusy   0x000593    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdcmd   0x0005a6    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdoff   0x0005bd    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdon   0x0005c2    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_passband_menu   0x000121    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_passband_string   0x000028    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_tolerance_menu   0x000137    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_tolerance_string   0x00003e    program     static D:\en0627_assignment.X\en0627_assignment.asm

main   0x0000d1    program     static D:\en0627_assignment.X\en0627_assignment.asm

mm_string   0x000065    program     static D:\en0627_assignment.X\en0627_assignment.asm

mult16x16   0x0004d4    program     static D:\en0627_assignment.X\en0627_assignment.asm

pass_string   0x000048    program     static D:\en0627_assignment.X\en0627_assignment.asm

perform_scan   0x0001ee    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_100_string   0x000358    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_10_string   0x000379    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_1_string   0x00039a    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_fail_string   0x0002fa    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_lower_passband_string   0x00033d    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_lower_tolerance_string   0x00034f    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_pass_string   0x0002b4    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_scan_string   0x0002ab    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_start_string   0x0002a2    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_100_string   0x0003bb    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_10_string   0x0003dc    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_1_string   0x0003fd    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_timeout_string   0x00031d    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_tolerance_string   0x0002d7    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_upper_passband_string   0x000336    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_upper_tolerance_string   0x000346    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan   0x00041e    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_end   0x000495    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_fail   0x00048f    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_pass   0x000424    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_string   0x00000f    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_timeout   0x000491    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lcd_bottom_line   0x0005b3    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lcd_top_line   0x0005ae    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lower_passband   0x000254    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lower_tolerance   0x000224    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_max_value   0x0001c6    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_min_value   0x0001cb    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_start   0x0001f7    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_upper_passband   0x00023b    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_upper_tolerance   0x00020b    program     static D:\en0627_assignment.X\en0627_assignment.asm

setup   0x000093    program     static D:\en0627_assignment.X\en0627_assignment.asm

software_error   0x00063c    program     static D:\en0627_assignment.X\en0627_assignment.asm

start_menu   0x00010d    program     static D:\en0627_assignment.X\en0627_assignment.asm

start_string   0x000004    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_1   0x0001ad    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_10   0x000194    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_100   0x00017b    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_100_string   0x00007e    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_10_string   0x000086    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_1_string   0x00008d    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check   0x0001df    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check_fail   0x0001ec    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check_pass   0x0001ed    program     static D:\en0627_assignment.X\en0627_assignment.asm

subt16x16   0x00055f    program     static D:\en0627_assignment.X\en0627_assignment.asm

success   0x0004b3    program     static D:\en0627_assignment.X\en0627_assignment.asm

testing_string   0x000014    program     static D:\en0627_assignment.X\en0627_assignment.asm

timeout   0x0004b0    program     static D:\en0627_assignment.X\en0627_assignment.asm

timeout_string   0x000056    program     static D:\en0627_assignment.X\en0627_assignment.asm

ultrasound   0x000496    program     static D:\en0627_assignment.X\en0627_assignment.asm

ults_end   0x0004cb    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd   0x0000f3    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_lower_passband_menu   0x000281    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_lower_tolerance_menu   0x000297    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_start_menu   0x00026d    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_upper_passband_menu   0x000276    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_upper_tolerance_menu   0x00028c    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_passband_menu   0x000116    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_passband_string   0x00001c    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_tolerance_menu   0x00012c    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_tolerance_string   0x000034    program     static D:\en0627_assignment.X\en0627_assignment.asm

wait_loop   0x0004ab    program     static D:\en0627_assignment.X\en0627_assignment.asm

Symbols - Sorted by Address

Name    Address   Location    Storage File

---------  ---------  ---------  --------- ---------

ISR   0x000004    program     static D:\en0627_assignment.X\en0627_assignment.asm

start_string   0x000004    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_string   0x00000f    program     static D:\en0627_assignment.X\en0627_assignment.asm

testing_string   0x000014    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_passband_string   0x00001c    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_passband_string   0x000028    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_tolerance_string   0x000034    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_tolerance_string   0x00003e    program     static D:\en0627_assignment.X\en0627_assignment.asm

pass_string   0x000048    program     static D:\en0627_assignment.X\en0627_assignment.asm

fail_string   0x00004f    program     static D:\en0627_assignment.X\en0627_assignment.asm

timeout_string   0x000056    program     static D:\en0627_assignment.X\en0627_assignment.asm

mm_string   0x000065    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_100_string   0x000069    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_10_string   0x000071    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_1_string   0x000078    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_100_string   0x00007e    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_10_string   0x000086    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_1_string   0x00008d    program     static D:\en0627_assignment.X\en0627_assignment.asm

setup   0x000093    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_00B7   0x0000b7    program     static D:\en0627_assignment.X\en0627_assignment.asm

main   0x0000d1    program     static D:\en0627_assignment.X\en0627_assignment.asm

TMR0_reset   0x0000df    program     static D:\en0627_assignment.X\en0627_assignment.asm

goto_menu   0x0000e1    program     static D:\en0627_assignment.X\en0627_assignment.asm

cycle_left   0x0000eb    program     static D:\en0627_assignment.X\en0627_assignment.asm

cycle_right   0x0000ef    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd   0x0000f3    program     static D:\en0627_assignment.X\en0627_assignment.asm

fin   0x0000fb    program     static D:\en0627_assignment.X\en0627_assignment.asm

error_check_lower   0x0000fe    program     static D:\en0627_assignment.X\en0627_assignment.asm

error_check_upper   0x000105    program     static D:\en0627_assignment.X\en0627_assignment.asm

start_menu   0x00010d    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_passband_menu   0x000116    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_passband_menu   0x000121    program     static D:\en0627_assignment.X\en0627_assignment.asm

upper_tolerance_menu   0x00012c    program     static D:\en0627_assignment.X\en0627_assignment.asm

lower_tolerance_menu   0x000137    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_100   0x000142    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_10   0x000155    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_1   0x000168    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_100   0x00017b    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_10   0x000194    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_1   0x0001ad    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_max_value   0x0001c6    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_min_value   0x0001cb    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check   0x0001d0    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check_fail   0x0001dd    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_error_check_pass   0x0001de    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check   0x0001df    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check_fail   0x0001ec    program     static D:\en0627_assignment.X\en0627_assignment.asm

sub_error_check_pass   0x0001ed    program     static D:\en0627_assignment.X\en0627_assignment.asm

perform_scan   0x0001ee    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_start   0x0001f7    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_upper_tolerance   0x00020b    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lower_tolerance   0x000224    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_upper_passband   0x00023b    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lower_passband   0x000254    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_start_menu   0x00026d    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_upper_passband_menu   0x000276    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_lower_passband_menu   0x000281    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_upper_tolerance_menu   0x00028c    program     static D:\en0627_assignment.X\en0627_assignment.asm

update_lcd_lower_tolerance_menu   0x000297    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_start_string   0x0002a2    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_scan_string   0x0002ab    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_pass_string   0x0002b4    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_tolerance_string   0x0002d7    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_fail_string   0x0002fa    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_timeout_string   0x00031d    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_upper_passband_string   0x000336    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_lower_passband_string   0x00033d    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_upper_tolerance_string   0x000346    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_lower_tolerance_string   0x00034f    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_100_string   0x000358    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_10_string   0x000379    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_add_1_string   0x00039a    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_100_string   0x0003bb    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_10_string   0x0003dc    program     static D:\en0627_assignment.X\en0627_assignment.asm

print_sub_1_string   0x0003fd    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan   0x00041e    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_pass   0x000424    program     static D:\en0627_assignment.X\en0627_assignment.asm

below_passband   0x000475    program     static D:\en0627_assignment.X\en0627_assignment.asm

above_passband   0x000482    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_fail   0x00048f    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_timeout   0x000491    program     static D:\en0627_assignment.X\en0627_assignment.asm

scan_end   0x000495    program     static D:\en0627_assignment.X\en0627_assignment.asm

ultrasound   0x000496    program     static D:\en0627_assignment.X\en0627_assignment.asm

wait_loop   0x0004ab    program     static D:\en0627_assignment.X\en0627_assignment.asm

timeout   0x0004b0    program     static D:\en0627_assignment.X\en0627_assignment.asm

success   0x0004b3    program     static D:\en0627_assignment.X\en0627_assignment.asm

got_result   0x0004b6    program     static D:\en0627_assignment.X\en0627_assignment.asm

ults_end   0x0004cb    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_04CC   0x0004cc    program     static D:\en0627_assignment.X\en0627_assignment.asm

mult16x16   0x0004d4    program     static D:\en0627_assignment.X\en0627_assignment.asm

add_y   0x00054c    program     static D:\en0627_assignment.X\en0627_assignment.asm

fin_y   0x000555    program     static D:\en0627_assignment.X\en0627_assignment.asm

add16x16   0x000556    program     static D:\en0627_assignment.X\en0627_assignment.asm

subt16x16   0x00055f    program     static D:\en0627_assignment.X\en0627_assignment.asm

div   0x000569    program     static D:\en0627_assignment.X\en0627_assignment.asm

div40bin   0x000569    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_setup   0x000572    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdbusy   0x000593    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdbsy1   0x000598    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdcmd   0x0005a6    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lcd_top_line   0x0005ae    program     static D:\en0627_assignment.X\en0627_assignment.asm

set_lcd_bottom_line   0x0005b3    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_clear   0x0005b8    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdoff   0x0005bd    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcdon   0x0005c2    program     static D:\en0627_assignment.X\en0627_assignment.asm

lcd_print   0x0005c7    program     static D:\en0627_assignment.X\en0627_assignment.asm

enable   0x0005d5    program     static D:\en0627_assignment.X\en0627_assignment.asm

delay   0x0005db    program     static D:\en0627_assignment.X\en0627_assignment.asm

delloop   0x0005dd    program     static D:\en0627_assignment.X\en0627_assignment.asm

deljmp   0x0005e3    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_ascii   0x0005e7    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_bcd   0x000604    program     static D:\en0627_assignment.X\en0627_assignment.asm

hex16_2_bcd_loop   0x000610    program     static D:\en0627_assignment.X\en0627_assignment.asm

adjDEC   0x00061b    program     static D:\en0627_assignment.X\en0627_assignment.asm

adjBCD   0x000628    program     static D:\en0627_assignment.X\en0627_assignment.asm

get_ascii   0x000631    program     static D:\en0627_assignment.X\en0627_assignment.asm

software_error   0x00063c    program     static D:\en0627_assignment.X\en0627_assignment.asm

_.org_1_0640   0x000640    program     static D:\en0627_assignment.X\en0627_assignment.asm

(mm)

(mm)

Appendix D: Graph of Results